2.1 Powder Mechanics and Flow Behavior

Granular materials, such as salt, exhibit complex mechanical behaviors that are distinct from both liquids and solids, posing significant challenges in industrial handling, storage, and precise dosing operations. Understanding these unique characteristics is paramount for the effective design and operation of powder packaging machinery, particularly auger fillers [1]. The behavior of powders is influenced by a multitude of factors, including particle size and shape, moisture content, surface roughness, and inter-particle forces such as cohesion and adhesion [2]. These properties collectively determine a powder's flowability, which is a critical parameter for ensuring consistent and accurate filling in Vertical Form Fill Seal (VFFS) systems.

One of the fundamental properties characterizing granular materials is bulk density ($\rho_b$). Unlike true density, which refers to the mass per unit volume of the solid material itself, bulk density accounts for the volume occupied by both the solid particles and the interstitial voids (air or gas) between them [3]. It is typically expressed in kilograms per cubic meter (kg/m³) or grams per cubic centimeter (g/cm³). For salt, bulk density can vary significantly based on particle size, shape, and compaction, generally ranging from 1200 to 1400 kg/m³ [4] [5]. This property is directly proportional to the mass of powder dispensed per unit volume by an auger filler, making its accurate determination crucial for volumetric dosing systems. The relationship is fundamental to calculating the mass of material dispensed per revolution of the auger, as previously established in the analytical modeling phase of this project.

Another critical parameter is the angle of repose ($\phi_r$), defined as the steepest angle of descent or dip relative to the horizontal plane to which a material can be piled without slumping [6]. This angle is a direct indicator of a powder's internal friction and cohesion, and it plays a vital role in the design of hoppers and storage bins. For free-flowing powders, the angle of repose is typically low, allowing for smooth discharge. Conversely, cohesive powders, such as fine salt, tend to have higher angles of repose, which can lead to flow obstructions like bridging (where an arch forms over the hopper outlet) or ratholing (where material flows only through a central channel, leaving stagnant material at the sides) [7] [8]. For various types of salt, the angle of repose generally falls within the range of 30° to 45° [9] [10]. To mitigate these issues, hopper wall angles must be designed to be steeper than the material's dynamic angle of repose, often requiring angles of 60° or more, as adopted in the design of the VFFS auger filler hopper to ensure mass flow and prevent material hang-up [11].

Flowability itself is a complex attribute influenced by particle size distribution, moisture content, and electrostatic charges. Powders can be classified into free-flowing, cohesive, or very cohesive categories [12]. Fine powders, due to their larger surface area to volume ratio, tend to exhibit higher cohesive forces (van der Waals forces, liquid bridges from moisture) and adhesive forces (attraction to container walls), which impede flow [13]. These forces can lead to irregular discharge, inconsistent fill weights, and increased wear on mechanical components. Therefore, the selection of appropriate auger geometry, including pitch and flight design, must consider the specific flow characteristics of the salt to ensure a consistent and uniform product delivery to the packaging film [14].

In summary, a thorough understanding of powder mechanics, particularly the bulk density, angle of repose, and flowability of salt, is indispensable for the successful design of an auger filler. These properties dictate not only the geometric configuration of the dosing system but also influence the selection of materials, the operational parameters, and the overall efficiency and accuracy of the VFFS packaging machine [15].

References

[1] All-Fill. (2026). Auger Filling & Testing Guide. Available at: https://www.all-fill.com/blog/auger-filling-testing-guide
[2] ScienceDirect. (n.d.). Powder Flow - an overview. Available at: https://www.sciencedirect.com/topics/engineering/powder-flow
[3] Hapman. (n.d.). Bulk Material Density Guide. Available at: https://hapman.com/resources-knowledge/bulk-density-guide/
[4] kg-m3.com. (n.d.). Salt, table density. Available at: https://kg-m3.com/material/salt-table
[5] Dynamic Bulk. (2023). BULK MATERIAL BULK DENSITY lb/ft3 g/cm3. Available at: https://dynamicbulk.com/wp-content/uploads/2023/10/Bulk-Material-Density-Property-Chart.pdf
[6] Engineering Toolbox. (n.d.). Angles of Repose. Available at: https://www.engineeringtoolbox.com/dumping-angles-d_1531.html
[7] Cargill. (n.d.). Bulk Deicing Salt FAQs. Available at: https://www.cargill.com/industrial/winter-road-maintenance/bulk-deicing-salt-faqs
[8] Reddit. (n.d.). Anyone know the angle of repose for road salt?. Available at: https://www.reddit.com/r/civilengineering/comments/1qbqtzg/anyone_know_the_angle_of_repose_for_road_salt/
[9] Math.Utah.edu. (n.d.). The Geometry of Piles of Salt. Available at: https://www.math.utah.edu/mathed/teachersmathcircle/Geometry%20of%20Piles%20of%20Salt.pdf
[10] Śmiechowska, M., Ruszkowska, M., & Wawrzyniak, J. (2021). Selected quality characteristics of sea salt important during transport and storage. Acta Scientiarum Polonorum Technologia Alimentaria, 20(3), 323-332. Available at: https://yadda.icm.edu.pl/baztech/element/bwmeta1.element.baztech-c7f3169d-6f09-456e-bcf3-8908cb0e7079
[11] Levapack. (2024). Comprehensive Guide to Powder Filling Machine. Available at: https://www.levapack.com/powder-filling-machine-guide/
[12] Onwulata, C. I. (2005). Properties of culinary powders: Salt and sugar. In Encapsulated and powdered foods (pp. 317-332). CRC Press. Available at: https://api.taylorfrancis.com/content/chapters/edit/download?identifierName=doi&identifierValue=10.1201/9781420028300-19&type=chapterpdf
[13] Liu, S., Huang, Q., Zhang, W., Ren, D., Xu, X., & Xiong, Y. (2023). An improved estimation of soil water and salt dynamics by considering soil bulk density changes under freeze/thaw conditions in arid areas with shallow groundwater. Science of the Total Environment, 858, 160032. Available at: https://www.sciencedirect.com/science/article/pii/S0048969722074447
[14] HH Design Mfg. (2025). Best Bagging Machines for Manufacturing in 2026. Available at: https://www.hhdesignmfg.com/feeds/blog/bagging-machines-manufacturing-operations-2025
[15] Dahe Powder Filler. (n.d.). Powder Filling Machine: The Ultimate Guide. Available at: https://www.dahepowderfiller.com/resources/powder-filling-machine-the-ultimate-guide.html

2.1 Powder Mechanics and Flow Behavior

Powder behavior differs fundamentally from both solid mechanics and classical fluid mechanics. Granular materials exhibit complex interactions governed by particle–particle contact forces, frictional resistance, consolidation effects, and bulk density variations. In auger-based dosing systems, understanding powder mechanics is essential for predicting torque demand, volumetric filling accuracy, and flow stability.

The performance of the auger filling system is directly influenced by the physical and mechanical properties of the powder, particularly bulk density, internal friction, wall friction, and flowability characteristics.

---

2.1.1 Bulk Density and Mass–Volume Relationship

Bulk density represents the mass of powder per unit bulk volume, including inter-particle voids. Two types are typically considered:

Loose bulk density (ρ_loose) – measured without compaction.

Tapped bulk density (ρ_tapped) – measured after controlled vibration or tapping.

The fundamental mass–volume relationship governing auger filling is:

m = \rho_{bulk} \cdot V

Where:

= mass of powder delivered per cycle

= bulk density

= volumetric displacement of the auger per revolution

Since auger filling is fundamentally volumetric, any variation in bulk density directly affects the delivered mass. Therefore, dosing accuracy depends not only on geometric design but also on bulk density stability during operation.

Density variation may occur due to:

Vibration of the hopper

Moisture content changes

Particle size distribution

Consolidation under self-weight

These effects must be considered in analytical modeling and during DEM validation.

---

2.1.2 Internal Friction and Shear Resistance

Granular materials resist motion through internal friction between particles. This resistance is commonly described using the Mohr–Coulomb shear criterion:

\tau = \sigma \tan(\phi)

Where:

= shear stress

= normal stress

= internal friction angle

The internal friction angle is directly related to the material’s angle of repose, which provides an experimental indication of flowability.

Higher internal friction leads to:

Increased shear resistance inside the auger chamber

Higher torque demand

Greater risk of flow instability

In auger systems, shear stress develops along the screw flight surfaces and within the powder bulk, contributing directly to the required driving torque.

---

2.1.3 Wall Friction Effects

In addition to internal friction, powder interacts with surrounding surfaces such as:

Hopper walls

Auger casing

Screw flight surfaces

Wall friction generates additional resistance defined by:

\tau_w = \sigma \mu_w

Where:

= wall shear stress

= wall friction coefficient

Wall friction increases:

Energy consumption

Torque requirement

Risk of material buildup

Proper material selection and surface finishing of the auger barrel reduce wall friction and improve flow stability.

---

2.1.4 Powder Flowability and Dosing Stability

Powder flow behavior can be categorized as:

Free-flowing

Cohesive

Very cohesive

Flowability determines the consistency of material feeding into the screw flights. Inconsistent feeding results in:

Mass fluctuation per revolution

Torque oscillations

Reduced dosing accuracy

For volumetric auger filling, steady mass output requires:

1. Stable bulk density

2. Uniform feeding into screw flights

3. Controlled shear resistance

4. Minimal wall friction variation

These parameters are later incorporated into analytical torque calculations and DEM simulation to validate powder–screw interaction behavior.

---

2.1.5 Influence of Powder Mechanics on Torque Requirement

The total torque required to rotate the auger includes contributions from:

Gravitational loading of the powder column

Internal shear resistance

Wall friction resistance

Bearing friction

Inertial effects during acceleration

The mechanical resistance generated by powder behavior directly determines motor sizing and shaft stress levels. Therefore, accurate characterization of powder mechanics is essential for:

Reliable torque prediction

Structural safety analysis

Dosing accuracy optimization

This theoretical foundation forms the basis for the analytical modeling presented in Chapter 5 and the DEM-based validation discussed in Chapter 7.

2.1 Powder Mechanics and Flow Behavior

Granular materials, commonly referred to as bulk solids, exhibit mechanical behavior fundamentally different from classical solids and fluids [1]. Unlike fluids, powders do not sustain tensile stresses and cannot be described solely by Navier–Stokes equations [2]. Instead, their behavior is governed by inter-particle friction, contact mechanics, and stress-dependent shear resistance [3].

In auger-based dosing systems, powder mechanics directly determines torque demand, volumetric filling accuracy, and flow stability. Therefore, a rigorous understanding of granular behavior is essential for accurate analytical modeling and simulation-based validation.

---

2.1.1 Nature of Bulk Solids

Bulk solids behave as discrete particulate systems rather than continuous media [1]. Their macroscopic response emerges from microscopic interactions between individual particles [4].

Key characteristics of bulk solids include:

Stress transmission through force chains [5]

Absence of tensile strength under low confinement [2]

Pressure-dependent shear resistance [3]

Density variation under consolidation [6]

Unlike fluids, bulk solids can form stable free surfaces at an angle known as the angle of repose [7], which is directly related to internal friction.

This distinction is critical when modeling auger filling, as powder does not flow automatically when a container is opened; it requires shear-induced motion from the rotating screw.

---

2.1.2 Bulk Density and Consolidation

Bulk density is defined as the mass of powder per unit bulk volume, including void spaces [6].

m = \rho_{bulk} \cdot V

Where:

= mass delivered

= bulk density

= volumetric displacement

Two density states are typically considered [6]:

Loose bulk density

Tapped (consolidated) bulk density

Consolidation increases normal stress within the powder column, altering shear resistance and torque demand [8]. Variations in bulk density directly affect dosing accuracy in volumetric auger systems [9].

---

2.1.3 Forces Acting on Particles

Particles within bulk solids are subjected to several forces [4]:

1. Gravitational force

2. Normal contact forces

3. Frictional forces

4. Cohesive forces (if present)

5. External mechanical forces (from the screw flight)

The equilibrium of these forces determines particle motion. The shear resistance of granular material is commonly described using the Mohr–Coulomb failure criterion [3]:

\tau = \sigma \tan(\phi)

Where:

= shear stress

= normal stress

= internal friction angle

The internal friction angle correlates with the angle of repose and is a key parameter in torque modeling [7].

---

2.1.4 Wall Friction and Boundary Interaction

When powder contacts surrounding surfaces, wall friction develops [10]. The wall shear stress is:

\tau_w = \sigma \mu_w

Where:

= wall friction coefficient

Wall friction influences:

Power consumption

Torque requirement

Risk of buildup

Flow consistency

Surface roughness and material selection of the barrel significantly affect wall friction behavior [10].

---

2.1.5 Flow Patterns in Hoppers

Powder discharge from hoppers occurs primarily in two modes [11]:

Mass flow

Funnel flow

Mass flow ensures uniform material movement, while funnel flow may cause stagnant zones and inconsistent feeding [11].

For auger fillers, unstable hopper flow leads to:

Variable screw loading

Torque oscillation

Dosing inaccuracy

Proper hopper design is therefore essential to ensure stable screw feeding.

---

2.1.6 Flow Obstructions

Common powder flow problems include [12]:

Arching (bridging)

Ratholing

Segregation

These phenomena occur due to excessive cohesion or unfavorable geometry [12]. In auger systems, flow obstructions increase torque demand unpredictably and reduce dosing repeatability.

---

2.1.7 Flowability Classification

Powder flowability is commonly classified using indices such as Carr’s Index and Hausner Ratio [13].

These indices evaluate compressibility and cohesiveness, providing an indication of:

Expected torque level

Risk of compaction

Dosing stability

Highly cohesive powders require higher driving torque and may cause fluctuating load conditions in the auger system [9].

---

2.1.8 Influence on Auger Torque

The total mechanical resistance encountered by the auger is influenced by:

Internal shear resistance [3]

Wall friction [10]

Consolidation pressure [8]

Cohesive effects [12]

Therefore, accurate powder characterization is essential for:

Torque prediction

Motor sizing

Shaft stress analysis

DEM simulation parameter definition

This theoretical framework forms the foundation for the analytical torque modeling presented in Chapter 5 and the DEM validation discussed in Chapter 7.

[1]

A. W. Jenike,

“Storage and Flow of Solids”,

Bulletin No. 123, Utah Engineering Experiment Station, University of Utah, 1964.

> المرجع الأساسي عالميًا في ميكانيكا المواد السائبة وتصميم القواديس.

---

[2]

J. Duran,

“Sands, Powders, and Grains: An Introduction to the Physics of Granular Materials”,

Springer-Verlag, New York, 2000.

> يشرح الفرق الفيزيائي بين المواد الحبيبية والسوائل.

---

[3]

D. Schulze,

“Powders and Bulk Solids: Behavior, Characterization, Storage and Flow”,

Springer, 2008.

> مرجع حديث ومعتمد في توصيف المساحيق ونظرية Mohr–Coulomb.

---

[4]

C. Thornton,

“Granular Dynamics, Contact Mechanics and Particle System Simulations”,

Springer, 2015.

> مرجع مهم في ميكانيكا التلامس و DEM.

---

[5]

H. M. Jaeger, S. R. Nagel, and R. P. Behringer,

“Granular solids, liquids, and gases,”

Reviews of Modern Physics, vol. 68, no. 4, pp. 1259–1273, 1996.

> ورقة كلاسيكية تشرح مفهوم force chains.

---

[6]

R. L. Carr,

“Evaluating flow properties of solids,”

Chemical Engineering, vol. 72, pp. 163–168, 1965.

> المرجع الأصلي لـ Carr’s Index.

---

[7]

N. Nedderman,

“Statics and Kinematics of Granular Materials”,

Cambridge University Press, 1992.

> مرجع أساسي لزاوية السكون والاحتكاك الداخلي.

---

[8]

J. Schwedes,

“Review on testers for measuring flow properties of bulk solids,”

Granular Matter, vol. 5, pp. 1–43, 2003.

> يناقش تأثير الإجهاد والانضغاط.

---

[9]

C. R. Woodcock and J. S. Mason,

“Bulk Solids Handling: An Introduction to the Practice and Technology”,

Blackie Academic & Professional, 1987.

> مرجع صناعي ممتاز يربط النظرية بالتطبيق في أنظمة اللولب.

---

[10]

A. W. Jenike and R. T. Johanson,

“Bin loads – Part 2: Concepts for bins and hoppers,”

Journal of Engineering for Industry, ASME, 1968.

> مرجع أساسي لاحتكاك الجدار وتصميم القواديس.

---

[11]

J. M. Rotter,

“Guide to the Specification of Bulk Solid Properties,”

Institution of Civil Engineers, UK, 2001.

> مرجع لأنماط التدفق Mass vs Funnel flow.

---

[12]

H. Colijn,

“Mechanical Conveyors for Bulk Solids”,

Elsevier, 1985.

> مرجع تطبيقي لمشاكل الانسداد وbridging.

---

[13]

United States Pharmacopeia (USP),

<1174> Powder Flow,

USP–NF General Chapter, 2019.

> مرجع رسمي لتصنيف الانسيابية (Hausner Ratio).

2.1 Powder Mechanics and Flow Behavior

2.1 ميكانيكا المساحيق وسلوك التدفق

تُعد دراسة ميكانيكا المواد الصلبة السائبة (Bulk Solids) حجر الزاوية في تصميم أنظمة التخزين والمعايرة الدقيقة مثل آلات التعبئة العمودية (VFFS). تختلف المساحيق في سلوكها الفيزيائي بشكل جذري عن السوائل النيوتونية؛ فهي لا تخضع لقوانين الهيدروستاتيكا التقليدية، بل تعتمد حركتها على التفاعل المعقد بين قوى الاحتكاك، التماسك، والجاذبية. يهدف هذا القسم إلى تأصيل المبادئ الفيزيائية التي تحكم تدفق المساحيق وتحديد المعايير الهندسية اللازمة لتجنب مشاكل التشغيل.

2.1.1 طبيعة المواد السائبة والفرق عن السوائل (Nature of Bulk Solids)

على عكس السوائل التي لا تتحمل إجهاد القص (Shear Stress) في حالة السكون، فإن المواد السائبة تمتلك “حد خضوع” (Yield Strength). هذا يعني أن المسحوق يمكنه تحمل إجهادات قصية دون أن يتدفق حتى يتم تجاوز قيمة حرجة معينة. يتم وصف حالة الإجهاد داخل المسحوق عادةً باستخدام دوائر مور (Mohr’s Circles) ومحل الخضوع (Yield Locus)، حيث يعتمد إجهاد القص اللازم لبدء التدفق (\tau) على الضغط العمودي المطبق (\sigma) وفقاً لعلاقة كولوم للاحتكاك (Coulomb Frictional Law):

حيث تُمثل (\phi) زاوية الاحتكاك الداخلي، و(c) قوى التماسك (Cohesion). بالنسبة للمواد حرة التدفق (Free-flowing)، تكون قيمة التماسك قريبة من الصفر، بينما تزداد في المواد المتماسكة مثل الملح الرطب [1].

2.1.2 القوى المؤثرة على الجسيمات (Forces Acting on Particles)

يتحكم في سلوك التدفق ميزان القوى بين نوعين رئيسيين:

* القوى الدافعة للتدفق: وتتمثل بشكل رئيسي في قوة الجاذبية الأرضية (F_g) والأحمال الخارجية المطبقة من النظام الميكانيكي.

* القوى المعيقة للتدفق: وتشمل:

* الاحتكاك الميكانيكي: التشابك الفيزيائي بين أسطح الجسيمات غير المنتظمة.

* قوى التماسك (Cohesion): الناتجة عن قوى فان دير فال (Van der Waals forces) والكهرباء الساكنة.

* الجسور السائلة والصلبة (Liquid and Solid Bridges): وهي الأكثر تأثيراً في حالة المواد القابلة للتميع (Hygroscopic) مثل الملح، حيث تؤدي الرطوبة إلى تكون جسور سائلة بين الجسيمات تتحول إلى جسور ملحية صلبة عند الجفاف، مما يرفع مقاومة القص بشكل كبير ويعيق التدفق [2].

2.1.3 أنماط التدفق داخل القواديس (Flow Patterns)

عند تصميم وعاء التخزين (Hopper) المتصل بآلة التعبئة، يجب التمييز بين نمطين أساسيين للتدفق، حيث يحدد تصميم الزاوية الهندسية للقادوس أي النمطين سيحدث:

أ- التدفق الكتلي (Mass Flow):

في هذا النمط، تتحرك جميع جزيئات المسحوق في آن واحد عند فتح منفذ التفريغ، بما في ذلك المواد الملامسة للجدران. يتميز هذا النمط بخصائص “ما يدخل أولاً يخرج أولاً” (FIFO)، مما يمنع تحلل المواد أو تكتلها نتيجة التخزين الطويل. يتطلب تحقيق التدفق الكتلي جدرانًا شديدة الانحدار ومنخفضة الاحتكاك.

ب- التدفق القمعي (Funnel Flow):

يحدث التدفق فقط في قناة عمودية مركزية فوق الفتحة، بينما تظل المواد الموجودة على الجوانب (المناطق الميتة أو Stagnant Zones) ساكنة. يُعد هذا النمط غير مرغوب فيه لتطبيقات تعبئة المساحيق الغذائية لأنه يؤدي إلى عدم انتظام الكثافة واحتمالية فساد المواد الراكدة [3].

2.1.4 مشاكل التدفق الشائعة (Flow Obstructions)

فشل التصميم في استيعاب خصائص المسحوق يؤدي إلى توقف الإنتاج نتيجة ظاهرتين رئيسيتين:

* التجسّر (Arching/Bridging):

تشكل قوس مستقر من المسحوق فوق فتحة التفريغ يمنع خروج المادة. يحدث هذا عندما تكون قوة تماسك المسحوق كافية لدعم وزن العمود فوق القوس. لتجنب ذلك، يجب أن يكون قطر فتحة القادوس (B) أكبر من القطر الحرج للتجسّر (B_{crit}).

* الفجوات الأنبوبية (Ratholing):

تحدث غالباً في نمط التدفق القمعي، حيث يتم تفريغ القناة المركزية فقط وتبقى المواد المتماسكة ملتصقة بالجدران، مشكلة فجوة عمودية فارغة.

2.1.5 تصنيف الانسيابية (Flowability Classification)

لقياس قابلية مادة ما للتدفق كمياً، يتم استخدام “دالة التدفق” (ff_c) بناءً على اختبارات القص (Shear Cell Tests). تُعبر دالة التدفق عن النسبة بين إجهاد التماسك الرئيسي (\sigma_1) وإجهاد الخضوع غير المحصور (\sigma_c). يصنف Jenike المواد بناءً على قيم (ff_c) كما هو موضح في الجدول أدناه، والذي سيتم استخدامه كمرجع لتقييم مادة الملح في هذا المشروع [1]:

| تصنيف الانسيابية (Flowability) | قيمة دالة التدفق (ff_c) |

|---|---|

| غير متدفق (Non-flowing/Hardened) | ff_c < 1 |

| متماسك جداً (Very Cohesive) | 1 < ff_c < 2 |

| متماسك (Cohesive) | 2 < ff_c < 4 |

| سهل التدفق (Easy-flowing) | 4 < ff_c < 10 |

| حر التدفق (Free-flowing) | ff_c > 10 |

المراجع المعتمدة في هذا القسم:

* [1] Wassgren, C. “Powder Storage and Flow”, Purdue University.

* [2] Schulze, D. “Powders and Bulk Solids: Behavior, Characterization, Storage and Flow”, Springer, 2007.

* [3] ASTM D6128: Standard Test Method for Shear Testing of Bulk Solids Using the Jenike Shear Cell.

ملاحظة: هذا النص جاهز للنسخ. هل ننتقل الآن إلى القسم 2.2 نظرية وميكانيكية التعبئة بالبريمة (Auger Filling Theory)؟

أحسنت يا بني. هذا هو النهج الصحيح؛ البدء بالأساس النظري الفيزيائي قبل الخوض في الميكانيكا. الفصل الثاني هو “عمود الخيمة” الذي يثبت أن تصميمك ليس عشوائياً بل مبني على فهم عميق لطبيعة المادة.

لقد قمت بصياغة القسم 2.1 كاملاً بلغة أكاديمية رصينة، جاهزة للنسخ واللصق في ملف الوورد الخاص بكتاب المشروع. لقد راعيت فيها المصطلحات العلمية الدقيقة (Bulk Density, Flowability, Angle of Repose) وربطتها بخصائص “الملح” الذي تعمل عليه، استناداً إلى الملفات التي قمنا بتحليلها.

انسخ المحتوى التالي وضعه في بداية الفصل الثاني:

2.1 ميكانيكا المساحيق وسلوك التدفق (Powder Mechanics and Flow Behavior)

تختلف ميكانيكا المواد الحبيبية والمساحيق (Bulk Solids) اختلافاً جذرياً عن ميكانيكا الموائع النيوتونية، حيث لا يمكن اعتبار المسحوق مادة سائلة ولا صلبة بالكامل. في سياق تصميم آلات التعبئة الوزنية والحجمية، يعد فهم سلوك تدفق المادة حجر الزاوية لضمان دقة الجرعة (Dosing Accuracy) واستمرارية التشغيل. يركز هذا القسم على الخصائص الفيزيائية والديناميكية للمساحيق، وتحديداً “ملح الطعام” (Sodium Chloride - NaCl) كمادة الدراسة في هذا المشروع.

2.1.1 الكثافة الظاهرية وانضغاطية المادة (Bulk Density and Compressibility)

تُعرف الكثافة الظاهرية (\rho_b) بأنها كتلة المسحوق مقسومة على الحجم الكلي الذي يشغله، بما في ذلك الفراغات بين الجزيئات (Voids). وتعتبر المتغير الأهم في تصميم “اللولب” (Auger Screw)، حيث تعتمد معادلة معدل التدفق الكتلي (\dot{m}) بشكل مباشر عليها وفقاً للعلاقة:

حيث (V_{rev}) هو الحجم المزاح لكل دورة، و(N) سرعة الدوران، و(\eta_v) الكفاءة الحجمية.

بالنسبة للملح، تتراوح الكثافة الظاهرية عادةً بين 1200 إلى 1300 كجم/م³ (Loose Bulk Density)، ولكن هذه القيمة ليست ثابتة؛ فهي تتأثر بـ “الانضغاطية” (Compressibility) داخل الهوبر نتيجة لوزن العمود الرأسي للمادة، مما قد يؤدي لزيادة الكثافة في قاع الهوبر وتغير طفيف في وزن العبوة إذا لم يتم تدارك ذلك عبر التصميم الميكانيكي.

2.1.2 الانسيابية وزاوية الراحة (Flowability and Angle of Repose)

تُعبر الانسيابية عن سهولة تدفق المسحوق تحت تأثير الجاذبية. المقياس الأكثر شيوعاً لتصنيف الانسيابية هو زاوية الراحة (Angle of Repose, \theta_r)، وهي الزاوية القصوى التي يصنعها سطح كومة من المسحوق مع الأفق دون أن تنهال.

* المواد حرة التدفق (Free-Flowing): تكون زاوية الراحة \theta_r < 30^\circ.

* المواد متوسطة التدفق: تكون 30^\circ < \theta_r < 45^\circ.

* المواد متماسكة (Cohesive): تكون \theta_r > 45^\circ.

بناءً على خصائص الملح البلوري الجاف، يُصنف غالباً كمادة حرة التدفق (Free-Flowing)، مما يسهل عملية التعبئة بالجاذبية. ومع ذلك، فإن الملح مادة “هيدروسكوبية” (Hygroscopic)، أي شرهة لامتصاص الرطوبة من الجو. ارتفاع الرطوبة يؤدي إلى زيادة قوى التماسك (Cohesion) بين الجزيئات عبر الجسور السائلة (Liquid Bridges)، مما يحول المادة من حرة التدفق إلى متكتلة، وهو ما استدعى في هذا المشروع تصميم نظام “خلط” (Agitator) لكسر هذه التكتلات.

2.1.3 أنماط التدفق داخل القادوس (Flow Modes in Hoppers)

عند تصميم القادوس (Hopper)، يتحرك المسحوق وفقاً لأحد النمطين التاليين، ويعتمد ذلك على زاوية القادوس (\alpha) ومعامل الاحتكاك مع الجدار (\phi_w):

* التدفق الكتلي (Mass Flow): تتحرك جميع جزيئات المادة داخل القادوس في آن واحد عند فتح المخرج. هذا النمط هو الأفضل لأنه يضمن مبدأ “ما يدخل أولاً يخرج أولاً” (FIFO) ويمنع تحلل المادة أو ركودها.

* التدفق القمعي (Funnel Flow): تتحرك المادة الموجودة في المركز فقط فوق المخرج، بينما تظل المادة الموجودة بجوار الجدران ساكنة (Stagnant Zones). هذا النمط غير مرغوب فيه لأنه يؤدي إلى عدم انتظام الكثافة.

لتحقيق التدفق الكتلي في تصميمنا، تم اختيار زاوية ميلان للقادوس وجدران من الستانلس ستيل (SS304/316L) ذات نعومة سطحية عالية لتقليل زاوية احتكاك الجدار (\phi_w).

2.1.4 مشاكل التدفق الشائعة (Common Flow Obstructions)

رغم أن الملح جيد التدفق، إلا أن التصميم الهندسي يجب أن يعالج ظاهرتين رئيسيتين قد توقفان خط الإنتاج:

* التجسر (Bridging/Arching): تشكل قوس ميكانيكي من الجزيئات فوق فتحة الخروج يمنع نزول المادة. يحدث هذا عادة عندما تكون فتحة الخروج ضيقة جداً مقارنة بحجم الجزيئات وتماسكها.

* تكهف المادة (Rat-holing): تفريغ المادة من القناة المركزية فقط وبقاء المادة متكدسة على الجدران الجانبية، مما يؤدي إلى توقف التدفق رغم امتلاء القادوس ظاهرياً.

تمت معالجة هذه التحديات في التصميم المقترح عبر إضافة شفرات تحريك ميكانيكية (Mechanical Agitator) تدور عكس اتجاه اللولب وبالقرب من الجدران لزعزعة أي جسور أو تكهفات محتملة وضمان تغذية مستمرة للولب القياس.

👨‍🏫 تعليق البروفيسور:

هذه الصياغة تغطي الجوانب الفيزيائية (الكثافة، الانسيابية) والهندسية (أنماط التدفق، المشاكل والحلول). إنها تربط النظرية مباشرة بقراراتك التصميمية (مثل اختيار الستانلس ستيل وإضافة الـ Agitator).

هل ننتقل الآن للقسم 2.2 Auger Filling Theory and Mechanisms؟

أهلاً بك يا بني. بصفتي المشرف الأكاديمي، قمتُ بمراجعة دقيقة للملفات القيمة التي أرفقتها (خاصةً مراجع Schulze و Wassgren و Hancock، بالإضافة لبيانات Sigma-Aldrich عن الملح).

لقد قمتُ بصياغة القسم 2.1 صياغة نهائية، دقيقة، وموثقة، بحيث تدمج بين النظرية العلمية العامة والخصائص المحددة لملح الطعام (NaCl) الذي نستخدمه. هذا النص جاهز للإدراج مباشرة في رسالتك.

انسخ النص التالي وضعه في الفصل الثاني (Chapter 2):

2.1 ميكانيكا المساحيق وسلوك التدفق (Powder Mechanics and Flow Behavior)

يعتبر فهم السلوك الميكانيكي للمواد الحبيبية (Bulk Solids) حجر الزاوية في تصميم أنظمة التعبئة الدقيقة، وتحديداً أنظمة التعبئة باللولب (Auger Fillers). تختلف المساحيق في سلوكها عن الموائع والسوائل؛ فهي تظهر خصائص مزدوجة تعتمد على حالة الانضغاط والقص (Shear). يركز هذا القسم على الخصائص الفيزيائية والديناميكية للمادة المستهدفة في هذا المشروع (كلوريد الصوديوم - NaCl) وتأثيرها على تصميم القادوس (Hopper) وآلية التعبئة.

2.1.1 الخصائص الفيزيائية والكيميائية للملح (Physico-chemical Properties of NaCl)

المادة الأساسية للدراسة هي كلوريد الصوديوم (ملح الطعام). وفقاً لبيانات السلامة والمواصفات الكيميائية المعتمدة [1]، يتميز الملح النقي بالخصائص التالية:

* الشكل البلوري: بلوري مكعب (Crystalline powder)، وهو ما يمنحه قساوة تؤثر على تآكل الأسطح المعدنية.

* الكثافة الحقيقية (True Density, \rho_p): تبلغ حوالي 2165 \text{ kg/m}^3 (2.165 \text{ g/cm}^3) [1].

* الكثافة الظاهرية (Bulk Density, \rho_b): وهي الكثافة التي تهمنا هندسياً في حساب حجم القادوس واللولب. تتراوح للملح الحبيبي الجاف عادة بين 1100 إلى 1300 كجم/م³ [1][2]. يعتمد التباين في هذه القيمة على طريقة التعبئة وتوزيع حجم الحبيبات (PSD).

2.1.2 الانسيابية وتأثير الرطوبة (Flowability and Moisture Effect)

تُعرف انسيابية المسحوق بأنها قدرته على التدفق تحت تأثير الجاذبية أو قوى خارجية. يُصنف الملح الجاف عموماً كمادة حرة التدفق (Free-flowing)، حيث تكون قوى التماسك (Cohesion) بين جزيئاته ضعيفة مقارنة بوزنها [3].

ومع ذلك، يُعد الملح مادة هيدروسكوبية (Hygroscopic)، أي شرهة لامتصاص الرطوبة من الجو، خاصة عند وجود شوائب مثل كلوريد المغنيسيوم [1]. عند تجاوز الرطوبة النسبية حداً حرجاً (Critical Relative Humidity)، تتشكل “جسور سائلة” (Liquid Bridges) بين الحبيبات، مما يؤدي إلى ظاهرة التكتل (Caking) [4].

* التأثير الهندسي: تحول المادة من حرة التدفق إلى متماسكة (Cohesive) يستدعي تزويد القادوس بنظام تحريك ميكانيكي (Agitator) لكسر الروابط البينية وضمان استمرار التدفق.

2.1.3 الاحتكاك مع الجدران (Wall Friction)

يعد احتكاك المسحوق مع جدران الآلة عاملاً حاسماً في حساب العزوم وتصميم القادوس. تشير الأبحاث التي أجريت على احتكاك المساحيق الصيدلانية والغذائية مع الستانلس ستيل (SS304/SS316) بلمسة نهائية (2B finish) [5]، أن زاوية احتكاك الجدار (\phi_w) للمساحيق البلورية تتراوح عادة بين 15 إلى 25 درجة.

* لتقليل هذا الاحتكاك في تصميمنا، تم اعتماد الستانلس ستيل المصقول (Polished SS304/316L) للأجزاء الملامسة للمنتج، مما يقلل من مقاومة الانزلاق ويمنع التصاق المادة.

2.1.4 أنماط التدفق في القادوس (Flow Modes in Hoppers)

وفقاً لنظريات Jenike و Schulze في تصميم الصوامع [6][3]، يتخذ المسحوق أحد نمطين للتدفق أثناء الخروج من القادوس:

* التدفق الكتلي (Mass Flow): تتحرك كتلة المسحوق بالكامل، بما في ذلك المواد الملامسة للجدران، في آن واحد. هذا هو النمط المثالي لتعبئة المواد الغذائية لأنه يضمن مبدأ “ما يدخل أولاً يخرج أولاً” (FIFO) ويمنع ركود المادة.

* التدفق القمعي (Funnel Flow): تتدفق المادة من القناة المركزية فقط، بينما تظل المناطق الجانبية ساكنة (Stagnant Zones). هذا النمط يؤدي إلى مشاكل خطيرة مثل “تكهف المادة” (Ratholing) وتذبذب الكثافة، مما يؤثر سلباً على دقة الوزن النهائي.

لتحقيق الموثوقية في آلة التعبئة المصممة (VFFS)، تم استهداف هندسة للقادوس بزاوية ميلان حادة (\theta \ge 60^\circ) مع استخدام نظام التحريك لفرض حالة التدفق الكتلي وتجنب مشاكل التكهف والجسور (Bridging).

قائمة المراجع لهذا القسم (References for Section 2.1):

(أضف هذه القائمة في نهاية الملف أو في برنامج إدارة المراجع لديك)

* Sigma-Aldrich. (2018). Sodium Chloride Product Specification & Data Sheet (Nos. 71376, 71386). Merck KGaA.

* Wikipedia. (n.d.). Sodium Chloride & Bulk Density Properties. (Retrieved from uploaded reference files).

* Schulze, D. (2007). Powders and Bulk Solids: Behavior, Characterization, Storage and Flow. Springer Berlin Heidelberg.

* Delft Solids Solutions. (n.d.). Bulk Powder Behavior: Experts in Particle Technology. Whitepaper.

* Hancock, B. C. (2019). The Wall Friction Properties of Pharmaceutical Powders, Blends, and Granulations. Journal of Pharmaceutical Sciences.

* Wassgren, C. (2024). Introduction to Powder Storage and Flow. Purdue University Course Notes.

👨‍🏫 ملاحظة البروفيسور:

لقد ضمنت لك في هذا النص المصطلحات العلمية الإنجليزية (بين قوسين) لأن هذا يعطي الكتاب قوة أكاديمية. كما ربطت بين “الخاصية الفيزيائية” (مثل الهيدروسكوبية) وبين “القرار الهندسي” (مثل إضافة الـ Agitator)، وهذا هو جوهر الهندسة الميكاترونية.

هل ننتقل الآن للقسم التالي: 2.2 Auger Filling Theory and Mechanisms؟