APPENDICES

Appendix A: Engineering Drawings (Mechanical)

• A.1. General Assembly (GA) Drawing: The full machine with a Bill of Materials (BOM).

Materials (BOM).

A.2. Sub-Assembly Drawings: Detailed views of the Hopper-Agitator unit and the Drive Train.

• A.3. Detailed Component Drawings: 2D dimensioned drawings of the Auger Screw (showing pitch and flight details), the Shaft, and the Discharge Nozzle.

A.4. Exploded View: To show the assembly sequence and maintenance access points.

Appendix B: Control and Mechatronics Documentation

B.1. Wiring Diagram: Electrical schematic showing the PLC/Microcontroller, Driver, and Motor connections.

B.2. Control Logic Flowchart: The algorithmic sequence for the "Fill-Stop-Seal" cycle.

B.3. HMI Screen Mockups: User interface design for entering target weight and speed.

B.4. Pseudo-code/Source Code: The core logic used for the motion control profile.

/*

* VFFS Machine Controller - Graduation Project

* * Features:

* 1. RMT Driver: Generates stepper pulses via Hardware (Non-blocking).

* 2. Hybrid Motion: RMT for speed (Blind Zone) + Manual for precision (Sensor Search).

* 3. Robust E-Stop: Kills RMT channels instantly via Fault Handler.

* 4. Optimized Core Usage: CPU 0 is free for precise PID calculations.

*/

#include <stdio.h>

#include <string.h>

#include <stdlib.h>

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "freertos/queue.h"

#include "freertos/semphr.h"

#include "freertos/event_groups.h"

#include "driver/gpio.h"

#include "driver/rmt.h" //

#include "esp_timer.h"

#include "esp_log.h"

#include "esp_rom_sys.h"

#include "esp_adc/adc_oneshot.h"

#include "mock_sensor.h"

// =========================================================

// ********** USER CONFIGURATION AREA **********

// =========================================================

#define SIMULATION_MODE     1

// PID & TEMP

#define HEATER_SETPOINT     150.0

#define PID_KP              20.0

#define PID_KI              0.5

#define PID_KD              0.0

#define PID_WINDOW_SIZE_MS  1000

#define TEMP_TOLERANCE      5.0

// HOPPER

#define HOPPER_EMPTY_THRESH 10

#define HOPPER_LOW_THRESH   30

// BAG & MOTION

#define BAG_LENGTH_STEPS    700

#define BLIND_ZONE_STEPS    500

#define MAX_SEARCH_STEPS    1000

#define AUGER_STEPS         800

// SPEED (RMT tick is usually 1us)

#define SPEED_FILM_US       1000

#define SPEED_AUGER_US      1000

// DELAYS

#define DELAY_JAWS_CLOSE    200

#define DELAY_FILL          200

#define DELAY_CUT           200

#define DELAY_OPEN          200

#define DELAY_CYCLE_END     200

// =========================================================

// PINS & DEFINITIONS

// =========================================================

// RMT CHANNELS

#define RMT_CH_AUGER        RMT_CHANNEL_0

#define RMT_CH_FILM         RMT_CHANNEL_1

// SPI

#define SPI_PIN_SCK         18

#define SPI_PIN_MISO        19

#define SPI_PIN_CS_1        21

// GPIOs

#define PIN_ESTOP           15

#define PIN_START_BTN       34

#define PIN_STOP_BTN        2

#define PIN_EYE_MARK        39

// Motors

#define PIN_AUGER_STEP      25

#define PIN_AUGER_DIR       26

#define PIN_AUGER_EN        27

#define PIN_FILM_STEP       32

#define PIN_FILM_DIR        33

#define PIN_FILM_EN         14

// Actuators

#define PIN_AGITATOR        13

#define PIN_HEATER_SSR      23

#define PIN_VALVE_HORIZ     4

#define PIN_VALVE_VERT      5

#define PIN_VALVE_CUTTER    12

// ADC

#define ADC_UNIT_ID         ADC_UNIT_1

#define ADC_CHAN_LEVEL      ADC_CHANNEL_7

// Logic

#define VALVE_ON            0

#define VALVE_OFF           1

#define AGIT_ON             1

#define AGIT_OFF            0

#define MOTOR_ENABLE        0

#define MOTOR_DISABLE       1

#define MOTOR_AUGER_ID      1

#define MOTOR_FILM_ID       2

// Events

#define BIT_SYSTEM_READY    (1 << 0)

#define BIT_AUTO_RUNNING    (1 << 1)

#define BIT_ESTOP_ACTIVE    (1 << 2)

#define BIT_TEMP_OK         (1 << 3)

// =========================================================

// HANDLES

// =========================================================

typedef struct {

uint8_t motor_id;

uint32_t steps;

uint32_t interval_us;

uint8_t direction;

bool use_sensor;

} MotionCommand_t;

EventGroupHandle_t sys_evt_group;

QueueHandle_t      motion_queue;

SemaphoreHandle_t  motion_done_sem;

SemaphoreHandle_t  mutex_vars;

TaskHandle_t th_ControlLoop = NULL;

TaskHandle_t th_PID = NULL;

TaskHandle_t th_HMI = NULL;

TaskHandle_t th_Motion = NULL;

adc_oneshot_unit_handle_t adc1_handle;

typedef struct {

double current_temp;

uint32_t bag_counter;

bool last_move_success;

} SystemData_t;

SystemData_t sys_data = { .current_temp = 25.0, .last_move_success = true };

// Forward Declarations

void Task_ControlLoop(void *pvParameters);

void Task_PID(void *pvParameters);

void Task_MotionController(void *pvParameters);

void Task_FaultHandler(void *pvParameters);

void Task_HMI(void *pvParameters);

// =========================================================

// RMT & HARDWARE SETUP

// =========================================================

void Init_RMT() {

rmt_config_t config = RMT_DEFAULT_CONFIG_TX(PIN_AUGER_STEP, RMT_CH_AUGER);

config.clk_div = 80; // 80MHz / 80 = 1MHz (1 tick = 1us) -> دقة ممتازة

rmt_config(&config);

rmt_driver_install(config.channel, 0, 0);

rmt_config_t config2 = RMT_DEFAULT_CONFIG_TX(PIN_FILM_STEP, RMT_CH_FILM);

config2.clk_div = 80;

rmt_config(&config2);

rmt_driver_install(config2.channel, 0, 0);

}

double Read_Temperature_Sensor() {

#if SIMULATION_MODE

return Read_Fake_Thermocouple(SPI_PIN_CS_1);

#else

// SPI Real Code

int v = 0;

gpio_set_level(SPI_PIN_CS_1, 0);

esp_rom_delay_us(10);

for (int i = 15; i >= 0; i--) {

gpio_set_level(SPI_PIN_SCK, 1); esp_rom_delay_us(10);

if (gpio_get_level(SPI_PIN_MISO)) v |= (1 << i);

gpio_set_level(SPI_PIN_SCK, 0); esp_rom_delay_us(10);

}

gpio_set_level(SPI_PIN_CS_1, 1);

if (v & 0x4) return -1.0;

return (v >> 3) * 0.25;

#endif

}

// =========================================================

// HELPERS

// =========================================================

void IRAM_ATTR isr_estop_handler(void* arg) {

BaseType_t xHigherPriorityTaskWoken = pdFALSE;

xEventGroupSetBitsFromISR(sys_evt_group, BIT_ESTOP_ACTIVE, &xHigherPriorityTaskWoken);

portYIELD_FROM_ISR(xHigherPriorityTaskWoken);

}

int get_hopper_level() {

int raw_val = 0;

if (adc_oneshot_read(adc1_handle, ADC_CHAN_LEVEL, &raw_val) == ESP_OK) {

return (raw_val * 100) / 4095;

}

return 0;

}

void SendMotionCommand(uint8_t id, uint32_t steps, uint32_t speed_us, uint8_t dir, bool use_sensor) {

MotionCommand_t cmd = { .motor_id = id, .steps = steps, .interval_us = speed_us, .direction = dir, .use_sensor = use_sensor };

xQueueSend(motion_queue, &cmd, portMAX_DELAY);

}

bool WaitForMotionResult() {

while(xSemaphoreTake(motion_done_sem, pdMS_TO_TICKS(100)) == pdFALSE) {

if (xEventGroupGetBits(sys_evt_group) & BIT_ESTOP_ACTIVE) return false;

}

bool success = true;

if (xSemaphoreTake(mutex_vars, pdMS_TO_TICKS(10))) {

success = sys_data.last_move_success;

xSemaphoreGive(mutex_vars);

}

return success;

}

// =========================================================

// TASKS

// =========================================================

// --- MOTION TASK (RMT ENABLED) ---

void Task_MotionController(void *pvParameters) {

MotionCommand_t cmd;

int dir_pin, en_pin;

rmt_channel_t rmt_chan;

for (;;) {

if (xQueueReceive(motion_queue, &cmd, portMAX_DELAY)) {

// 1. Setup

if (cmd.motor_id == MOTOR_AUGER_ID) {

dir_pin = PIN_AUGER_DIR; en_pin = PIN_AUGER_EN; rmt_chan = RMT_CH_AUGER;

} else {

dir_pin = PIN_FILM_DIR; en_pin = PIN_FILM_EN; rmt_chan = RMT_CH_FILM;

}

gpio_set_level(dir_pin, cmd.direction);

gpio_set_level(en_pin, MOTOR_ENABLE);

esp_rom_delay_us(50);

bool move_ok = true;

// 2. Execution Logic

if (cmd.use_sensor) {

// === HYBRID MODE: RMT for Blind Zone + Manual for Search ===

printf("[MOTION] Hybrid Pull: RMT (Blind) -> Manual (Search)\n");

// A) RMT Phase: Blind Zone (Fast & CPU Free)

if (BLIND_ZONE_STEPS > 0) {

// Create RMT items buffer

size_t size = BLIND_ZONE_STEPS * sizeof(rmt_item32_t);

rmt_item32_t* items = (rmt_item32_t*) malloc(size);

if (items) {

for (int i = 0; i < BLIND_ZONE_STEPS; i++) {

// High/Low duration = half interval

items[i] = (rmt_item32_t){{{cmd.interval_us/2, 1, cmd.interval_us/2, 0}}};

}

// Send and Wait

rmt_write_items(rmt_chan, items, BLIND_ZONE_STEPS, true);

rmt_wait_tx_done(rmt_chan, portMAX_DELAY); // Wait for HW to finish

free(items);

}

}

// Check Safety after RMT

if (xEventGroupGetBits(sys_evt_group) & BIT_ESTOP_ACTIVE) move_ok = false;

// B) Manual Phase: Search Zone (Precise)

if (move_ok) {

bool mark_found = false;

int step_pin = (cmd.motor_id == MOTOR_AUGER_ID) ? PIN_AUGER_STEP : PIN_FILM_STEP;

uint32_t remaining_search = MAX_SEARCH_STEPS - BLIND_ZONE_STEPS;

for (uint32_t i = 0; i < remaining_search; i++) {

if (xEventGroupGetBits(sys_evt_group) & BIT_ESTOP_ACTIVE) { move_ok = false; break; }

gpio_set_level(step_pin, 1); esp_rom_delay_us(cmd.interval_us / 2);

gpio_set_level(step_pin, 0); esp_rom_delay_us(cmd.interval_us / 2);

if (gpio_get_level(PIN_EYE_MARK) == 0) { // Found!

mark_found = true;

printf("[MOTION] Mark Found in Search Zone.\n");

break;

}

}

if (!mark_found && move_ok) {

printf("[MOTION] Error: Mark Missed!\n");

move_ok = false;

}

}

} else {

// === PURE RMT MODE (Auger) ===

// Generate all pulses via hardware

size_t size = cmd.steps * sizeof(rmt_item32_t);

rmt_item32_t* items = (rmt_item32_t*) malloc(size);

if (items) {

for (int i = 0; i < cmd.steps; i++) {

items[i] = (rmt_item32_t){{{cmd.interval_us/2, 1, cmd.interval_us/2, 0}}};

}

rmt_write_items(rmt_chan, items, cmd.steps, true);

// While RMT works, we wait. If E-Stop happens, FaultHandler kills RMT.

// We just need to wait for completion or abort.

while (rmt_wait_tx_done(rmt_chan, 10 / portTICK_PERIOD_MS) != ESP_OK) {

if (xEventGroupGetBits(sys_evt_group) & BIT_ESTOP_ACTIVE) {

move_ok = false; break;

}

}

free(items);

}

}

gpio_set_level(en_pin, MOTOR_DISABLE);

if (xSemaphoreTake(mutex_vars, pdMS_TO_TICKS(10))) {

sys_data.last_move_success = move_ok;

xSemaphoreGive(mutex_vars);

}

xSemaphoreGive(motion_done_sem);

}

}

}

// --- PID TASK (Precise due to RMT) ---

void Task_PID(void *pvParameters) {

// Setup SPI

gpio_set_direction(SPI_PIN_CS_1, GPIO_MODE_OUTPUT);

gpio_set_direction(SPI_PIN_SCK, GPIO_MODE_OUTPUT);

gpio_set_direction(SPI_PIN_MISO, GPIO_MODE_INPUT);

gpio_set_level(SPI_PIN_CS_1, 1);

double temp_current = 0.0, pid_output = 0.0;

double error = 0.0, last_error = 0.0, integral = 0.0;

int64_t window_start = esp_timer_get_time() / 1000;

for (;;) {

int64_t now_ms = esp_timer_get_time() / 1000;

int64_t in_window = (now_ms - window_start);

if (in_window >= PID_WINDOW_SIZE_MS) {

window_start = now_ms;

in_window = 0;

temp_current = Read_Temperature_Sensor();

if (temp_current == -1.0) temp_current = sys_data.current_temp; // Fallback

if (xSemaphoreTake(mutex_vars, pdMS_TO_TICKS(10))) {

sys_data.current_temp = temp_current;

xSemaphoreGive(mutex_vars);

}

printf("[PID] T:%.1f C | Pwr:%.0f%%\n", temp_current, (pid_output/PID_WINDOW_SIZE_MS)*100.0);

error = HEATER_SETPOINT - temp_current;

integral += error;

if (integral > 100.0) integral = 100.0;

if (integral < -100.0) integral = -100.0;

double output_val = (PID_KP * error) + (PID_KI * integral) + (PID_KD * (error - last_error));

last_error = error;

if (output_val > PID_WINDOW_SIZE_MS) output_val = PID_WINDOW_SIZE_MS;

if (output_val < 0) output_val = 0;

pid_output = output_val;

if (temp_current > (HEATER_SETPOINT - TEMP_TOLERANCE)) xEventGroupSetBits(sys_evt_group, BIT_TEMP_OK);

else xEventGroupClearBits(sys_evt_group, BIT_TEMP_OK);

}

if (in_window < pid_output) gpio_set_level(PIN_HEATER_SSR, 1);

else gpio_set_level(PIN_HEATER_SSR, 0);

vTaskDelay(pdMS_TO_TICKS(20));

}

}

// --- LOGIC & HMI TASKS (UNCHANGED) ---

void Task_ControlLoop(void *pvParameters) {

vTaskDelay(pdMS_TO_TICKS(1000));

xEventGroupSetBits(sys_evt_group, BIT_SYSTEM_READY);

for (;;) {

EventBits_t bits = xEventGroupWaitBits(sys_evt_group, BIT_AUTO_RUNNING, pdFALSE, pdTRUE, pdMS_TO_TICKS(500));

if (bits & BIT_AUTO_RUNNING) {

if (!(xEventGroupGetBits(sys_evt_group) & BIT_TEMP_OK)) {

printf("[SYS] Paused: Waiting for Temp...\n");

vTaskDelay(pdMS_TO_TICKS(1000)); continue;

}

int level = get_hopper_level();

if (level < HOPPER_EMPTY_THRESH) {

printf("[ALARM] Hopper EMPTY!\n");

xEventGroupClearBits(sys_evt_group, BIT_AUTO_RUNNING); continue;

} else if (level < HOPPER_LOW_THRESH) {

printf("[WARNING] Low Material (%d%%).\n", level);

}

printf("[CYCLE] 1. Pulling Film...\n");

SendMotionCommand(MOTOR_FILM_ID, 0, SPEED_FILM_US, 1, true);

if (!WaitForMotionResult()) {

printf("[FAULT] Film Error.\n");

xEventGroupClearBits(sys_evt_group, BIT_AUTO_RUNNING); continue;

}

printf("[CYCLE] 2. Jaws Closing...\n");

gpio_set_level(PIN_VALVE_HORIZ, VALVE_ON);

gpio_set_level(PIN_VALVE_VERT, VALVE_ON);

vTaskDelay(pdMS_TO_TICKS(DELAY_JAWS_CLOSE));

printf("[CYCLE] 3. Filling...\n");

gpio_set_level(PIN_AGITATOR, AGIT_ON);

vTaskDelay(pdMS_TO_TICKS(DELAY_FILL));

SendMotionCommand(MOTOR_AUGER_ID, AUGER_STEPS, SPEED_AUGER_US, 1, false);

WaitForMotionResult();

printf("[CYCLE] >> CUT\n");

gpio_set_level(PIN_VALVE_CUTTER, VALVE_ON);

vTaskDelay(pdMS_TO_TICKS(DELAY_CUT));

gpio_set_level(PIN_VALVE_CUTTER, VALVE_OFF);

printf("[CYCLE] 4. Open Jaws\n");

gpio_set_level(PIN_VALVE_VERT, VALVE_OFF);

gpio_set_level(PIN_AGITATOR, AGIT_OFF);

printf("[CYCLE] 6. Release Horizontal\n");

gpio_set_level(PIN_VALVE_HORIZ, VALVE_OFF);

vTaskDelay(pdMS_TO_TICKS(DELAY_OPEN));

if (xSemaphoreTake(mutex_vars, pdMS_TO_TICKS(10))) {

sys_data.bag_counter++;

printf("[DATA] Bags: %lu\n", sys_data.bag_counter);

xSemaphoreGive(mutex_vars);

}

vTaskDelay(pdMS_TO_TICKS(DELAY_CYCLE_END));

} else {

vTaskDelay(pdMS_TO_TICKS(500));

}

}

}

void Task_HMI(void *pvParameters) {

int last_start = 0, last_stop = 0;

for (;;) {

int start = gpio_get_level(PIN_START_BTN);

if (start == 1 && last_start == 0) {

EventBits_t status = xEventGroupGetBits(sys_evt_group);

if (status & BIT_ESTOP_ACTIVE) {

printf("[HMI] REJECTED: E-STOP Active!\n");

} else if ((status & BIT_TEMP_OK) == 0) {

printf("[HMI] REJECTED: Warming Up... (Wait for %.1f C)\n", HEATER_SETPOINT);

} else {

printf("[HMI] Start Accepted\n");

xEventGroupSetBits(sys_evt_group, BIT_AUTO_RUNNING);

}

}

last_start = start;

int stop = gpio_get_level(PIN_STOP_BTN);

if (stop == 1 && last_stop == 0) {

printf("[HMI] Stop Received\n");

xEventGroupClearBits(sys_evt_group, BIT_AUTO_RUNNING);

}

last_stop = stop;

vTaskDelay(pdMS_TO_TICKS(50));

}

}

// --- FAULT HANDLER (CRITICAL: RMT STOP) ---

void Task_FaultHandler(void *pvParameters) {

EventBits_t bits;

for (;;) {

bits = xEventGroupWaitBits(sys_evt_group, BIT_ESTOP_ACTIVE, pdFALSE, pdFALSE, portMAX_DELAY);

if (bits & BIT_ESTOP_ACTIVE) {

printf("\n!!! E-STOP TRIGGERED !!!\n");

// 1. Kill RMT Immediately (Hardware Stop)

rmt_tx_stop(RMT_CH_AUGER);

rmt_tx_stop(RMT_CH_FILM);

// 2. Suspend Tasks

if (th_ControlLoop) vTaskSuspend(th_ControlLoop);

if (th_PID) vTaskSuspend(th_PID);

if (th_Motion) vTaskSuspend(th_Motion);

// 3. Kill Outputs

gpio_set_level(PIN_AUGER_EN, MOTOR_DISABLE);

gpio_set_level(PIN_FILM_EN, MOTOR_DISABLE);

gpio_set_level(PIN_HEATER_SSR, 0);

gpio_set_level(PIN_VALVE_HORIZ, VALVE_OFF);

gpio_set_level(PIN_VALVE_VERT, VALVE_OFF);

gpio_set_level(PIN_VALVE_CUTTER, VALVE_OFF);

gpio_set_level(PIN_AGITATOR, AGIT_OFF);

xEventGroupClearBits(sys_evt_group, BIT_AUTO_RUNNING);

while(gpio_get_level(PIN_ESTOP) == 0) vTaskDelay(pdMS_TO_TICKS(50));

printf("[SYS] E-STOP Released. Resetting...\n");

// Respawn

if (th_Motion) { vTaskDelete(th_Motion); th_Motion = NULL; }

xTaskCreatePinnedToCore(Task_MotionController, "MotionTask", 4096, NULL, 5, &th_Motion, 0);

if (th_ControlLoop) { vTaskDelete(th_ControlLoop); th_ControlLoop = NULL; }

xTaskCreatePinnedToCore(Task_ControlLoop, "ControlTask", 4096, NULL, 2, &th_ControlLoop, 1);

if (th_PID) { vTaskDelete(th_PID); th_PID = NULL; }

xTaskCreatePinnedToCore(Task_PID, "PIDTask", 2048, NULL, 3, &th_PID, 0);

xEventGroupClearBits(sys_evt_group, BIT_ESTOP_ACTIVE);

}

}

}

void app_main() {

sys_evt_group = xEventGroupCreate();

mutex_vars = xSemaphoreCreateMutex();

motion_queue = xQueueCreate(5, sizeof(MotionCommand_t));

motion_done_sem = xSemaphoreCreateBinary();

adc_oneshot_unit_init_cfg_t init_config1 = { .unit_id = ADC_UNIT_ID };

adc_oneshot_new_unit(&init_config1, &adc1_handle);

adc_oneshot_chan_cfg_t config = { .bitwidth = ADC_BITWIDTH_DEFAULT, .atten = ADC_ATTEN_DB_12 };

adc_oneshot_config_channel(adc1_handle, ADC_CHAN_LEVEL, &config);

gpio_config_t io_conf = {};

io_conf.intr_type = GPIO_INTR_DISABLE;

// Heater

io_conf.mode = GPIO_MODE_INPUT_OUTPUT;

io_conf.pin_bit_mask = (1ULL<<PIN_HEATER_SSR);

io_conf.pull_down_en = 0; io_conf.pull_up_en = 0;

gpio_config(&io_conf);

// Outputs

io_conf.mode = GPIO_MODE_OUTPUT;

gpio_set_level(PIN_VALVE_HORIZ, VALVE_OFF);

gpio_set_level(PIN_VALVE_VERT, VALVE_OFF);

gpio_set_level(PIN_VALVE_CUTTER, VALVE_OFF);

gpio_set_level(PIN_AGITATOR, AGIT_OFF);

io_conf.pin_bit_mask = (1ULL<<PIN_AUGER_EN) | (1ULL<<PIN_AUGER_DIR) | // No STEP here (RMT handles it)

(1ULL<<PIN_FILM_EN) | (1ULL<<PIN_FILM_DIR) |

(1ULL<<PIN_AGITATOR) |

(1ULL<<PIN_VALVE_HORIZ) | (1ULL<<PIN_VALVE_VERT) | (1ULL<<PIN_VALVE_CUTTER) |

(1ULL<<SPI_PIN_CS_1) | (1ULL<<SPI_PIN_SCK);

gpio_config(&io_conf);

// Buttons

io_conf.mode = GPIO_MODE_INPUT;

io_conf.pin_bit_mask = (1ULL<<PIN_START_BTN) | (1ULL<<PIN_STOP_BTN);

io_conf.pull_up_en = 0; io_conf.pull_down_en = 0;

gpio_config(&io_conf);

// Sensors

io_conf.pin_bit_mask = (1ULL<<PIN_EYE_MARK) | (1ULL<<SPI_PIN_MISO);

io_conf.pull_up_en = 1; io_conf.pull_down_en = 0;

gpio_config(&io_conf);

// E-Stop

io_conf.intr_type = GPIO_INTR_NEGEDGE;

io_conf.pin_bit_mask = (1ULL<<PIN_ESTOP);

io_conf.pull_up_en = 0;

gpio_config(&io_conf);

gpio_install_isr_service(0);

gpio_isr_handler_add(PIN_ESTOP, isr_estop_handler, NULL);

//

Init_RMT(); // تهيئة  RMT

#if SIMULATION_MODE

Start_Mock_Engine();

#endif

// Core Distribution

xTaskCreatePinnedToCore(Task_FaultHandler, "FaultTask", 2048, NULL, 5, NULL, 0);

xTaskCreatePinnedToCore(Task_MotionController, "MotionTask", 4096, NULL, 4, &th_Motion, 0);

xTaskCreatePinnedToCore(Task_PID, "PIDTask", 2048, NULL, 3, &th_PID, 0);

xTaskCreatePinnedToCore(Task_ControlLoop, "ControlTask", 4096, NULL, 2, &th_ControlLoop, 1);

xTaskCreatePinnedToCore(Task_HMI, "HMITask", 2048, NULL, 1, &th_HMI, 1);

printf("System Ready (RMT Accelerated).\n");

}

Appendix C: Simulation and Data Reports

C.1. Mesh Sensitivity Analysis Data: Tables showing how results converged with different mesh sizes.

| Platform | Mesh Type | Element Size | Elements | Max Stress (MPa) |
| --- | --- | --- | --- | --- |
| SOLIDWORKS | Coarse | - | 19779 | 21.3 |
| SOLIDWORKS | Medium | - | 50377 | 85.37 |
| SOLIDWORKS | Fine | - | 351350 | 127.3 |
| ANSYS | Coarse | 10 mm | 52340 | 62.1 |
| ANSYS | Medium | 5 mm | 118450 | 85.92 |
| ANSYS | Fine | 2.5 mm | 554032 | 86.39 |

C.2. Material Datasheets: Technical properties of the powder used (e.g., Flour, Milk Powder) and the Stainless Steel grade.

Table C.2 presents the bulk density values and flow properties for the target powders in this study:

| Material | Bulk Density (kg/m³) | AoR (°) | Cohesion (Pa) | Flow Function |
| --- | --- | --- | --- | --- |
| Coffee Powder | 250-400 | 30-40 | 150-300 | Cohesive |
| Granulated Sugar | 700-880 | 30-36 | 50-100 | Easy-flowing |
| Wheat Flour | 560-600 | 38-45 | 300-500 | Very cohesive |
| Fine Table Salt | 1200-1400 | 32-35 | 100-200 | Easy-flowing |
| Powdered Detergent | 300-600 | 30-45 | 200-400 | Cohesive |
| Milk Powder | 200-720 | 35-44 | 250-450 | Cohesive |

Table C.2: Bulk Density and Flow Properties of Target Powders

C.3. Extended Simulation Plots: High-resolution color contours of von Mises stress, total deformation, and thermal gradients.

’

Appendix D: Calculations and Tables

D.1. Motor Sizing Spreadsheet: Detailed torque-speed calculations.

Optimal Motor Selection Table - VFFS Auger Filler

I. Basic Design Requirements

| Item | Value | Unit | Notes |
| --- | --- | --- | --- |
| Required Design Torque (Tdesign) | 1.14 | N·m | With SF = 3.0 |
| Design Torque with Reserve (Tdesign,total) | 1.332 | N·m | ×1.2 for unexpected conditions |
| Required Speed at Auger | 150 | RPM | For filling in 0.6 seconds |
| Maximum Angular Acceleration | 300 | rad/s² | For rapid response |

II. Comparative Motor Options

| Specification | Non Acceptable Option | Acceptable Option | Current Option |
| --- | --- | --- | --- |
| Model | MSMF012L1U1 | MSMF022L1U1 | MSMF042L1U1 |
| Manufacturer | Panasonic | Panasonic | Panasonic |
| Power | 100 W | 200 W | 400 W |
| Rated Torque (Trated) | 0.32 N·m | 0.64 N·m | 1.27 N·m |
| Peak Torque (Tpeak) | 0.95 N·m | 1.91 N·m | 3.82 N·m |
| Rated Speed | 3000 RPM | 3000 RPM | 3000 RPM |
| Maximum Speed | 6000 RPM | 6000 RPM | 6000 RPM |
| Rotor Inertia (Jrotor) | 0.051 ×10-4 kg·m² | 0.14 ×10-4 kg·m² | 0.27 ×10-4 kg·m² |
| Weight | 0.47 kg | 0.78 kg | 1.2 kg |
| Flange Dimensions | 40 mm | 60 mm | 60 mm |
| Protection Rating | IP67 | IP67 | IP67 |
| Approximate Price | $150 | $220 | $400 |

III. Gearbox Compatibility

| Item | Non Acceptable Option | Acceptable Option | Current Option |
| --- | --- | --- | --- |
| Proposed Gear Ratio (i) | 5:1 | 10:1 | 20:1 (Current) |
| Required Servo Torque | 1.48 N·m | 0.74 N·m | 0.37 N·m |
| Available Servo Torque | 0.32 N·m | 0.64 N·m | 1.27 N·m |
| Safety Margin (SFactual) | 0.216 | 1 | 3.5 |
| Evaluation | Not safe | Weak | Very safe |
| Speed at Auger | 600 RPM | 300 RPM | 150 RPM |
| Required Modification | Electronic speed reduction | None | None |

IV. Practical Application Compatibility

| Criterion | Non Acceptable Option | Acceptable Option | Current Option (MSMF042L1U1) |
| --- | --- | --- | --- |
| Stopping Precision | High | High | Very High |
| Acceleration Response | Very Fast | Fast | Relatively Slow |
| Power Consumption | Low (30W) | Medium (60W) | High (120W) |
| Cooling Requirement | Natural | Natural | Additional Fan |
| Lifespan | Long (10+ years) | Very Long | Very Long |
| Maintenance Ease | Easy | Easy | Easy |
| Spare Parts Availability | Bad | Not good | Excellent |

Comprehensive Torque Equations Table - VFFS Auger Filler

I. Fundamental Material Torques

| No. | Torque Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Material Weight Torque | Tw=m×g×rm | m = Material mass (0.5 kg) g = Gravity acceleration (9.81 m/s²) rm= Mean radius =ro+ri2 | 0.093 | N·m | Basic torque required to lift the material. |
| 2 | Axial Friction Torque | Tf=μ×m×g×rm | μ = Friction coefficient (0.3 for fine salt) | 0.028 | N·m | Material resistance to movement. |
| 3 | Wall Friction Torque | Twall=μw×fn×ro | μw = Wall friction coefficient (0.2-0.4) Fn= Normal force =m×g×cos⁡(α) ro = Outer radius (0.03 m) | 0.015 | N·m | Friction of the auger flight against the tube wall. |
| 4 | Internal Shear Torque | Ts=τ×A×rm | τ = Material shear stress (100-500 Pa for salt) A = Shear area =π×Ds×Lf | 0.05 | N·m | Material resistance to deformation. |

II. Dynamic Torques

| No. | Torque Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | Auger Inertia Torque | Tj,auger=Jauger×α | Jauger=12×mauger×(ro2+ri2) α = Angular acceleration = ∆ω∆t | 0.008 | N·m | Impactful during startup and stopping. |
| 6 | Material Inertia Torque | Tj,material=Jmaterial×α | Jmaterial=12×m×(ro2+ri2) | 0.004 | N·m | Accounts for the mass of the accelerating material. |
| 7 | Centrifugal Torque | Tc=m×w2×rm×μ×ro | ω = Angular velocity (rad/s) ω=2π×N60 | 0.012 | N·m | Increases significantly at high speeds. |
| 8 | Vibration Torque | Tv=mv×AV×Rm | mv = Vibrator mass (0.1 kg) av = Vibration acceleration (2-5 m/s²) | 0.006 | N·m | Applicable if a vibration motor is utilized. |

III. Mechanical Torques

| No. | Torque Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | Bearing Friction Torque | Tbearing=12×μb×Fr×db | μb = Bearing friction coefficient (0.001-0.005) Fr = Radial load (N) db = Bearing diameter (0.02 m) | 0.0002 | N·m | Very small but critical for precision. |
| 10 | Gear Loss Torque | Tgear,loss=Tin×(1-n) | Tin = Input torque n = Gear efficiency (0.94) | Variable | N·m | Represents roughly 6% of lost power. |
| 11 | Coupling Loss Torque | Tcoupling=μc×Fc×rc | μc= Coupling friction coefficient (0.1) Fc = Coupling compression force rc = Coupling radius | 0.003 | N·m | Minor losses occurring at the connections. |
| 12 | Air Resistance Torque | Tair=12×ρair×cd×A×v2×ro | ρair = Air density (1.225 kg/m³) cd = Drag coefficient (0.5-1.0) A = Exposed area v = Linear velocity | 0.001 | N·m | Extremely negligible, typically ignored. |

IV. Final Design Torques

| No. | Torque Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | Total Raw Torque | Traw=Tw+Tf+Twall+Ts+Tj,total+TC+Tbearing+Tcoupling | Sum of all preceding torques | 0.37 | N·m | Value before applying any safety factors. |
| 14 | Required Design Torque | Tdesign=Traw×SF | SF = Safety Factor (3.0 as specified) | 1.14 | N·m | The primary value used for design. |
| 15 | Design Torque with Reserve | Tdesign,total =Tdesign×1.2 | 1.2 = Reserve factor for unexpected operational conditions | 1.37 | N·m | Ensures components are never overstressed. |

V. Servo Conversion Equations

| No. | Torque Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 16 | Required Servo Torque | Tservo=Tdesign,totali×n | i = Gear ratio (20) n = Gear efficiency (0.94) | 0.033 | N·m | Torque required directly at the servo shaft. |
| 17 | Servo Peak Torque (Acceleration) | Tservo,peak=Tservo+Jtotal,×αmaxi×n | Jtotal = Total system inertia αmax = Maximum acceleration | 0.045 | N·m | Torque spike during maximum acceleration. |
| 18 | Actual Safety Margin | SFactual=Tservo,ratedTservo | Tservo,rated= Servo rated torque (1.27 N·m) | 38.5 | - | Excessively high (confirms the motor is oversized). |
| 19 | Gear Output Torque | Toutput=T servo,rate×i×n | - | 23.9 | N·m | Total available torque at the auger shaft. |

Comprehensive Volumetric Equations Table - Auger Filler VFFS

First: Basic Auger Geometry Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Annular Cross-Sectional Area | A=π4​×(D2-Ds2) | D = Outer auger diameter (0.06 m) Ds = Central shaft diameter (0.016 m) | 0.00267 | m² | Effective area for material transfer |
| 2 | Outer Radius | ro​=D2​ | D = Outer diameter | 0.03 | m | For circular calculations |
| 3 | Inner Radius | ri=Ds2 | Ds= Shaft diameter | 0.008 | m | For circular calculations |
| 4 | Mean Radius | rm=ro+ri2 | - | 0.019 | m | For torque and force calculations |
| 5 | Radial Flight Height | hf=ro-ri=D-Ds3 | - | 0.022 | m = 22 mm | Helical chamber depth |
| 6 | Outer Circumference | Co=π×D | - | 0.1885 | m | For linear speed calculations |
| 7 | Inner Circumference | Ci=π×Ds | - | 0.0503 | m | For inner speed calculations |

Second: Auger Volume Calculations Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 8 | Theoretical Volume per Pitch | Vpitch,theory=A×p=π4(D2-Ds2)×p | p = Pitch = Distance between flights (0.06 m) | 0.000160 | m³ = 160 cm³ | Without considering voids |
| 9 | Effective Volume per Pitch | Vpitch,eff=ηv×Vpitch,theory =ηv×A×p | ηv = Volumetric filling efficiency (0.6 for fine salt) | 0.000096 | m³ = 96 cm³ | Considering voids between material particles |
| 10 | Total Auger Volume | Vauger,total = Vpitch,eff×n=ηv×A×p×n | n = Number of effective pitches (4.5) | 0.000432 | m³ = 432 cm³ | Volume of material inside the auger |
| 11 | Volume per Complete Revolution | Vrev = Vpitch,eff=ηv×A×p | - | 0.000096 | m³ | Same as Equation 9 |
| 12 | Volume per Degree of Rotation | Vdeg = Vpitch,eff360°=ηv×A×p360 | - | 2.67 × 10⁻⁷ | m³/° | For precise Servo control |
| 13 | Volume per Pulse | Vpulse = Vpitch,effPPR=ηv×A×p8,388,608 | PPR = Encoder resolution (23-bit = 8,388,608) | 1.14 × 10⁻¹¹ | m³/pulse | Ultra-high precision control |

Third: Mass & Flow Calculations Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 14 | Mass per Pitch | mpitch =V pitch,eff×ρb =ηv×A×p×ρb | ρb= Bulk density of material (1300 kg/m³ for salt) | 0.125 | kg = 125 g | Theoretical mass per pitch |
| 15 | Mass per Revolution | mrev=mpitch =ηv×A×p×ρb | - | 0.125 | kg | Same as Equation 14 |
| 16 | Total Auger Mass | mauger,total = Vauger,total×ρb=ηv×A×p×n×ρb | - | 0.562 | kg = 562 g | Maximum mass inside the auger |
| 17 | Volumetric Flow Rate | Qv = Vpitch,eff×N =ηv×A×p×N | N = Angular speed (rev/sec) | Variable | m³/s | Material volume/time |
| 18 | Mass Flow Rate | m=Qv ×ρb =ηv×A×p×N×ρb | - | Variable | kg/s | Material mass/time |
| 19 | Target Dose Mass | mdose,target = 1.0 | Determined from customer requirements | 1.0 | kg = 1000 g | Target to be achieved |
| 20 | Required Revolutions per Dose | Ndose=mdose,targetmrev =mdose,targetηv×A×p×ρb | - | 8.0 | rev | To obtain 1000 g |
| 21 | Required Pitches per Dose | ndose=mdose,targetmpitch =mdose,targetηv×A×p×ρb | - | 8.0 | pitch | Same as Equation 20 |
| 22 | Required Rotation Angle | θdose=Ndose×360=mdose,target×360ηv×A×p×ρb | - | 2880 | degree | For Servo Position Control |
| 23 | Required Filling Time | NdoseNauger=mdose,target×360ηv×A×p×ρb×Nauger | Nauger = Auger speed (rev/sec) | 0.6 | s | For synchronization with bag cycle |

Fourth: Spinner Plate Volume Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 24 | Circular Plate Area | Adisc=π4×Ddisc2 | Ddisc = Plate diameter (0.10 m) | 0.00785 | m² | Total area of the plate |
| 25 | Sloped Area | Aslop= π×(rdisc2-rflat2) | rdisc = Plate radius (0.05 m) rflat = Flat area radius (0.034775 m) | 0.00415 | m² | Effective sliding area |
| 26 | Material Volume on Plate | Vdisc=Adisc×hlayer= π4×Ddisc2×hlayer | hlayer= Material layer thickness (0.01-0.02 m) | 0.0000785 | m³ = 78.5 cm³ | For continuous feeding |
| 27 | Material Mass on Plate | mdisc=Vdisc×ρb= Adisc×hlayer×ρb | - | 0.102 | kg = 102 g | Reserve above the auger |
| 28 | Effective Slope Angle | αeff= arctan⁡(hdiscrdisc-rflat) | hdisc= Plate edge height (0.01 m) | 55 | degree | Affects flow speed |

Fifth: Hopper Volume Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 29 | Cylindrical Hopper Volume | Vhopper,cyl=π4×Dh2×Hh | Dh = Hopper diameter (0.3 m) Hh= Hopper height (0.4 m) | 0.0283 | m³ = 28.3 L | For temporary storage |
| 30 | Conical Hopper Volume | Vhopper,cyl=π12×Hc×(Dt2+Dt×Db+Db2) | Hc = Cone height Dt = Top diameter Db = Bottom diameter | Variable | m³ | For smooth feeding |
| 31 | Material Volume in Hopper | Vmaterial,hopper = ηfill×Vhopper | ηfill = Hopper fill factor (0.8-0.9) | Variable | m³ | Less than geometric volume |
| 32 | Material Mass in Hopper | mhopper=Vmaterial,hopper ×ρb | - | Variable | kg | For production planning |
| 33 | Complete Emptying Time | tempty= mhopperm=Vmaterial,hopperQv | - | Variable | s | Between two hopper refills |

Sixth: Bag Volume Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 34 | Rectangular Bag Volume | Vbag=ω×h×t | ω = Width (0.15 m) h= Height (0.20 m) t = Thickness/Depth (0.08 m) | 0.0024 | m³ = 2.4 L | Internal bag volume |
| 35 | Available Product Volume | Vproduct= Vbag×ηfill,bag=ω×h×t×ηfill,bag | ηfill,bag= Bag fill factor (0.7-0.8) | 0.0018 | m³ = 1.8 L | Considering air |
| 36 | Maximum Bag Mass | mbag,max=ρb×Vproduct | - | 2.34 | kg | For safety verification |
| 37 | Required Film Length | Lfilm= 2×ω+t+Lseal | Lseal= Seal margin (0.02 m) | 0.48 | m = 480 mm | Per bag |
| 38 | Required Film Width | wfilm = h + Lseal,top} +Lseal,bottom | Lseal,bottom = Top/bottom seal margin | 0.24 | m = 240 mm | Roll width |

Seventh: Pressure & Stress Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 39 | Hydrostatic Column Pressure | Ph= ρb×g×hcolumn | hcoulmn= Material column height above auger (0.2 m) | 2550 | Pa = 2.55 kPa | Pressure on the auger |
| 40 | Vertical Force from Pressure | Fh=Ph×A=ρb×g×hcolumn×π4×Ddisc2 | - | 20.0 | N | Needed for structural design |
| 41 | Shear Stress in Material | τ=FshearAshear=T2π×rm2×hf | T = Applied torque | Variable | Pa | For viscous materials |
| 42 | Compressibility Factor | K =∆P∆VVo | ∆P = Pressure change ∆V = Volume change | Variable | Pa | For compressible materials |

Eighth: Speed & Time Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 43 | Angular Velocity | ω=2π×N=2π×nRPM60 | N = Speed (rev/sec) nRPM = Speed (rev/min) | 15.7 | rad/s | For unit conversion |
| 44 | Linear Material Velocity | vmaterial=ω×rm×tan⁡(∅) | ∅ = Internal friction angle (25-35°) | Variable | m/s | Actual transfer speed |
| 45 | Complete Filling Cycle Time | Tcycle=tfill+tsettle+tseal+tcut+tfrad | tsettle = Settling time tseal = Sealing time tcut = Cutting time tfrad = Feeding time | 1.71 | s | For total productivity |
| 46 | Productivity | n=60Tcycle | - | 35 | bag/min | Production target |

Ninth: Correction & Calibration Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 47 | Volumetric Correction Factor | Cv=mactualmtheory=mactualηv×A×p×ρb×n | mactual = Measured actual mass | Variable | - | Determined experimentally |
| 48 | Actual Bulk Density | ρactual=mactualVauger,total=mactualηv×A×p×n | - | Variable | kg/m³ | May differ from 1300 |
| 49 | Actual Filling Efficiency | ηactual=mactualρb×A×p×n | - | Variable | - | For precise calibration |
| 50 | Corrected Number of Revolutions | Ncorrected=Ntheory×1Cv=mtargetmactual×Ntheory | - | Variable | rev | For precise control |

Summary of Key Values

| Item | Value | Unit | Main Equation |
| --- | --- | --- | --- |
| Annular Area (A) | 0.00267 | m² | A=π4​⋅(D2-Ds2) |
| Effective Volume per Pitch | 96 | cm³ | ηv×A×p |
| Total Auger Volume | 432 | cm³ | ηv×A×p×n |
| Mass per Pitch | 125 | g | ηv×A×p×ρb |
| Total Auger Mass | 562 | g | ηv×A×p×ρb×n |
| Required Revolutions for 1000g | 8.0 | rev | mtargetηv×A×p×ρb |
| Rotation Angle for 1000g | 2880 | degree | Ndose× 360 |
| Filling Time | 0.6 | s | NdoseNauger |

| Required Design Torque (Tdesign) | 1.14 | N·m | With SF = 3.0 |
| --- | --- | --- | --- |
| Design Torque with Reserve (Tdesign,total) | 1.332 | N·m | ×1.2 for unexpected conditions |
| Required Speed at Auger | 150 | RPM | For filling in 0.6 seconds |
| Maximum Angular Acceleration | 300 | rad/s² | For rapid response |

D.2. Bearing and Fastener Selection Tables: Reference data used to justify the selection of specific industrial parts.

II. Controller Selection (ESP32 vs. PLC)

| Feature | Custom ESP32 Architecture | Traditional Industrial PLC |
| --- | --- | --- |
| Processing Power | Dual-Core 240 MHz (Parallel) | Single-Core 50-100 MHz (Cyclic) |
| Motion Control | Native RMT (Hardware-level) | Needs High-Speed IO Modules |
| Cost Efficiency | Highly Cost-Effective | High License & Hardware Costs |
| Connectivity | Native WiFi/UART (Industry 4.0) | Requires Communication Gateways |

Industrial Hardware Specifications

| Component | Part Number/Type | Technical Justification |
| --- | --- | --- |
| Thermal Sensor | K-Type + MAX6675 | High range (up to $1024^\circ\text{C}$) & SPI Digital interface. |
| Eye-mark Sensor | KS-C2 (NPN) | High-speed contrast detection for registration. |
| Heater Relay | Zero-Crossing SSR 40A | Eliminates EMI noise during PID switching. |
| Logic Isolation | Optocoupler Relay Module | Protects ESP32 (3.3V) from 24V industrial spikes. |
| Power Supply | MeanWell 24V, 14.6A | Provides stable power for motors and actuators. |

D.3. Error Analysis Calculations: The statistical math used to compare analytical vs. simulation results

II. Simulation vs. Requirement Analysis

| Metric | Design Requirement | Simscape Result | Percentage Error / Status |
| --- | --- | --- | --- |
| Setpoint | $150^\circ\text{C}$ | $150^\circ\text{C}$ | 0% (Exact match) |
| Settling Time | $< 300\text{ s}$ | $225.3\text{ s}$ | Within Bounds (PASSED) |
| Max Overshoot | $< 1\%$ | $0.304\%$ | Highly Stable (PASSED) |
| Steady-State Error | $\pm 1^\circ\text{C}$ | $\pm 0.45^\circ\text{C}$ | Improved precision |
| Derivative Filter ($N$) | Stable ($ | 1-NT_s | <1$) |