Comprehensive Volumetric Equations Table - Auger Filler VFFS

First: Basic Auger Geometry Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Annular Cross-Sectional Area | A=π4​⋅(D2-Ds2) | D = Outer auger diameter (0.06 m) Ds= = Central shaft diameter (0.016 m) | 0.00267 | m² | Effective area for material transfer |
| 2 | Outer Radius | ro​=D2​ | D = Outer diameter | 0.03 | m | For circular calculations |
| 3 | Inner Radius | ri=Ds2 | Ds= = Shaft diameter | 0.008 | m | For circular calculations |
| 4 | Mean Radius | rm=ro+ri2 | - | 0.019 | m | For torque and force calculations |
| 5 | Radial Flight Height | hf=ro-ri=D-Ds3 | - | 0.022 | m = 22 mm | Helical chamber depth |
| 6 | Outer Circumference | Co=π×D | - | 0.1885 | m | For linear speed calculations |
| 7 | Inner Circumference | Ci=π×Ds | - | 0.0503 | m | For inner speed calculations |

Second: Auger Volume Calculations Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 8 | Theoretical Volume per Pitch | Vpitch,theory=A×p=π4(D2-Ds2)×p | p = Pitch = Distance between flights (0.06 m) | 0.000160 | m³ = 160 cm³ | Without considering voids |
| 9 | Effective Volume per Pitch | Vpitch,eff=ηv×Vpitch,theory=ηv×A×p | ηv = Volumetric filling efficiency (0.6 for fine salt) | 0.000096 | m³ = 96 cm³ | Considering voids between material particles |
| 10 | Total Auger Volume | Vauger,total = Vpitch,eff×n=ηv×A×p×n | $n$ = Number of effective pitches (4.5) | 0.000432 | m³ = 432 cm³ | Volume of material inside the auger |
| 11 | Volume per Complete Revolution | Vrev = Vpitch,eff =ηv×A×p | - | 0.000096 | m³ | Same as Equation 9 |
| 12 | Volume per Degree of Rotation | Vdeg = Vpitch,eff360°=ηv×A×p360 | - | 2.67 × 10⁻⁷ | m³/° | For precise Servo control |
| 13 | Volume per Pulse | Vpulse = Vpitch,effPPR=ηv×A×p8,388,608 | PPR = Encoder resolution (23-bit = 8,388,608) | 1.14 × 10⁻¹¹ | m³/pulse | Ultra-high precision control |

Third: Mass & Flow Calculations Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 14 | Mass per Pitch | mpitch =V pitch,eff×ρb=ηv×A×p×ρb | ρb= Bulk density of material (1300 kg/m³ for salt) | 0.125 | kg = 125 g | Theoretical mass per pitch |
| 15 | Mass per Revolution | mrev=mpitch=ηv×A×p×ρb | - | 0.125 | kg | Same as Equation 14 |
| 16 | Total Auger Mass | mauger,total = Vauger,total×ρb=ηv×A×p×n×ρb | - | 0.562 | kg = 562 g | Maximum mass inside the auger |
| 17 | Volumetric Flow Rate | Qv = Vpitch,eff×N=ηv×A×p×N | N = Angular speed (rev/sec) | Variable | m³/s | Material volume/time |
| 18 | Mass Flow Rate | m=Qv ×ρb=ηv×A×p×N×ρb | - | Variable | kg/s | Material mass/time |
| 19 | Target Dose Mass | mdose,target = 1.0 | Determined from customer requirements | 1.0 | kg = 1000 g | Target to be achieved |
| 20 | Required Revolutions per Dose | Ndose=mdose,targetmrev =mdose,targetηv×A×p×ρb | - | 8.0 | rev | To obtain 1000 g |
| 21 | Required Pitches per Dose | ndose=mdose,targetmpitch =mdose,targetηv×A×p×ρb | - | 8.0 | pitch | Same as Equation 20 |
| 22 | Required Rotation Angle | θdose=Ndose×360=mdose,target×360ηv×A×p×ρb | - | 2880 | degree | For Servo Position Control |
| 23 | Required Filling Time | NdoseNauger=mdose,target×360ηv×A×p×ρb×Nauger | Nauger = Auger speed (rev/sec) | 0.6 | s | For synchronization with bag cycle |

Fourth: Spinner Plate Volume Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 24 | Circular Plate Area | Adisc=π4×Ddisc2 | Ddisc = Plate diameter (0.10 m) | 0.00785 | m² | Total area of the plate |
| 25 | Sloped Area | Aslop=π×(rdisc2-rflat2) | rdisc = Plate radius (0.05 m) rflat = Flat area radius (0.034775 m) | 0.00415 | m² | Effective sliding area |
| 26 | Material Volume on Plate | Vdisc=Adisc×hlayer=π4×Ddisc2×hlayer | hlayer= Material layer thickness (0.01-0.02 m) | 0.0000785 | m³ = 78.5 cm³ | For continuous feeding |
| 27 | Material Mass on Plate | mdisc=Vdisc×ρb=Adisc×hlayer×ρb | - | 0.102 | kg = 102 g | Reserve above the auger |
| 28 | Effective Slope Angle | αeff=arctan⁡(hdiscrdisc-rflat) | hdisc= Plate edge height (0.01 m) | 55 | degree | Affects flow speed |

Fifth: Hopper Volume Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 29 | Cylindrical Hopper Volume | Vhopper,cyl=π4×Dh2×Hh | Dh = Hopper diameter (0.3 m) Hh= Hopper height (0.4 m) | 0.0283 | m³ = 28.3 L | For temporary storage |
| 30 | Conical Hopper Volume | Vhopper,cyl=π12×Hc×(Dt2+Dt×Db+Db2) | Hc = Cone height Dt = Top diameter Db = Bottom diameter | Variable | m³ | For smooth feeding |
| 31 | Material Volume in Hopper | Vmaterial,hopper =ηfill×Vhopper | ηfill = Hopper fill factor (0.8-0.9) | Variable | m³ | Less than geometric volume |
| 32 | Material Mass in Hopper | mhopper=Vmaterial,hopper ×ρb | - | Variable | kg | For production planning |
| 33 | Complete Emptying Time | tempty= mhopperm=Vmaterial,hopperQv | - | Variable | s | Between two hopper refills |

Sixth: Bag Volume Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 34 | Rectangular Bag Volume | Vbag=ω×h×t | ω = Width (0.15 m) h= Height (0.20 m) t = Thickness/Depth (0.08 m) | 0.0024 | m³ = 2.4 L | Internal bag volume |
| 35 | Available Product Volume | Vproduct=Vbag×ηfill,bag=ω×h×t×ηfill,bag | ηfill,bag= Bag fill factor (0.7-0.8) | 0.0018 | m³ = 1.8 L | Considering air |
| 36 | Maximum Bag Mass | mbag,max=ρb×Vproduct | - | 2.34 | kg | For safety verification |
| 37 | Required Film Length | Lfilm=2×ω+t+Lseal | Lseal= Seal margin (0.02 m) | 0.48 | m = 480 mm | Per bag |
| 38 | Required Film Width | wfilm = h + Lseal,top} +Lseal,bottom | Lseal,bottom = Top/bottom seal margin | 0.24 | m = 240 mm | Roll width |

Seventh: Pressure & Stress Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 39 | Hydrostatic Column Pressure | Ph=ρb×g×hcolumn | $h_{column}$ = Material column height above auger (0.2 m) | 2550 | Pa = 2.55 kPa | Pressure on the auger |
| 40 | Vertical Force from Pressure | Fh=Ph×A=ρb×g×hcolumn×π4×Ddisc2 | - | 20.0 | N | Needed for structural design |
| 41 | Shear Stress in Material | τ=FshearAshear=T2π×rm2×hf | T = Applied torque | Variable | Pa | For viscous materials |
| 42 | Compressibility Factor | K =∆P∆VVo | ∆P = Pressure change ∆V = Volume change | Variable | Pa | For compressible materials |

Eighth: Speed & Time Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 43 | Angular Velocity | ω=2π×N=2π×nRPM60 | N = Speed (rev/sec) nRPM = Speed (rev/min) | 15.7 | rad/s | For unit conversion |
| 44 | Linear Material Velocity | vmaterial=ω×rm×tan⁡(∅) | ∅ = Internal friction angle (25-35°) | Variable | m/s | Actual transfer speed |
| 45 | Complete Filling Cycle Time | Tcycle=tfill+tsettle+tseal+tcut+tfrad | tsettle = Settling time tseal = Sealing time tcut = Cutting time tfrad = Feeding time | 1.71 | s | For total productivity |
| 46 | Productivity | n=60Tcycle | - | 35 | bag/min | Production target |

Ninth: Correction & Calibration Equations

| No. | Equation Name | Equation | Symbols & Meanings | Calculated Value | Unit | Engineering Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 47 | Volumetric Correction Factor | Cv=mactualmtheory=mactualηv×A×p×ρb×n | mactual = Measured actual mass | Variable | - | Determined experimentally |
| 48 | Actual Bulk Density | ρactual=mactualVauger,total=mactualηv×A×p×n | - | Variable | kg/m³ | May differ from 1300 |
| 49 | Actual Filling Efficiency | ηactual=mactualρb×A×p×n | - | Variable | - | For precise calibration |
| 50 | Corrected Number of Revolutions | Ncorrected=Ntheory×1Cv=mtargetmactual×Ntheory | - | Variable | rev | For precise control |

Summary of Key Values for Your Case

| Item | Value | Unit | Main Equation |
| --- | --- | --- | --- |
| Annular Area (A) | 0.00267 | m² | $\frac{\pi}{4}(D^2 - D_s^2)$ |
| Effective Volume per Pitch | 96 | cm³ | ηv×A×p |
| Total Auger Volume | 432 | cm³ | ηv×A×p×n |
| Mass per Pitch | 125 | g | ηv×A×p×ρb |
| Total Auger Mass | 562 | g | ηv×A×p×ρb×n |
| Required Revolutions for 1000g | 8.0 | rev | mtargetηv×A×p×ρb |
| Rotation Angle for 1000g | 2880 | degree | Ndose× 360 |
| Filling Time | 0.6 | s | NdoseNauger |