|  | Simulation of Auger Date: Thursday, April 9, 2026 Designer: Solidworks Study name: Auger static analysis Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Interaction Information 5 Mesh information 5 Sensor Details 5 Resultant Forces 6 Beams 6 Study Results 7 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Auger Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet18; Solid Body; Mass:1.17609 kg Volume:0.00014702 m^3 Density:7,999.56 kg/m^3 Weight:11.5257 N; C:\Graduation graduate\Auger Filler of Powder VFFS\2. The second stage\AUGER FILLER SYSTEM\analysis\solidworks analysis\Auger analysis\Auger.SLDPRT Apr 9 11:38:58 2026.

# Study Properties Study name; Auger static analysis. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Graduation graduate\Auger Filler of Powder VFFS\2. The second stage\AUGER FILLER SYSTEM\analysis\solidworks analysis\Auger analysis).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; AISI 304. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.06807e+08 N/m^2. Tensile strength:; 5.17017e+08 N/m^2. Elastic modulus:; 1.9e+11 N/m^2. Poisson's ratio:; 0.29. Mass density:; 8,000 kg/m^3. Shear modulus:; 7.5e+10 N/m^2. Thermal expansion coefficient:; 1.8e-05 /Kelvin.; SolidBody 1(Fillet18)(0.Auger (الطول الكلي 444سم)). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 2 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -47.5094; -1.3853; -11.4987; 48.9007. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Torque-1; Entities:; 1 face(s). Type:; Apply torque. Value:; 20 N.m.. Gravity-1; Reference:; Front Plane. Values:; 0 0 9.81. Units:; m/s^2..

# Connector Definitions No Data

# Interaction Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 5.41099 mm. Minimum element size; 0.27055 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 104572. Total Elements; 63131. Maximum Aspect Ratio; 71.497. % of elements with Aspect Ratio < 3; 99.3. Percentage of elements with Aspect Ratio > 10; 0.0475. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:57. Computer name:; HASSAN.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -47.5094; -1.3853; -11.4987; 48.9007. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.580267; -0.0988879; -9.34634; 9.36486. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 9.295e+00N/m^2 Node: 42563; 9.174e+07N/m^2 Node: 96867. Auger-Auger static analysis-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 36; 6.361e-01mm Node: 42551. Auger-Auger static analysis-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 2.029e-10 Element: 16819; 3.776e-04 Element: 53648. Auger-Auger static analysis-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Max von Mises Stress; 2.254e+00 Node: 96867; 2.225e+07 Node: 42563. Auger-Auger static analysis-Factor of Safety-Factor of Safety1.

# Conclusion