Auger (Screw):

A — Basic Parameters

Material handled: Fine salt

Target bag weight: 0.5 kg

Production rate: 15 bags/min (t_fill = 2 s)

Bulk density: 1300 kg/m³

Material: Stainless Steel (SS316)

Hopper capacity: 20 kg

B — Main Geometry

Outer diameter (D): 60 mm

Outer radius: 30 mm

Shaft diameter: 16 mm

Shaft radius: 8 mm

Pitch (p): 60 mm

Number of turns: 4

Flighted (effective) length: 228 mm

Total length: 350 mm (40 mm inlet + 228 mm flighted + 80 mm coupling zone)

C — Flight and Section Dimensions

Flight thickness: 4 mm

Radial height (D - D_shaft)/2 = 22 mm

Clearance (tip to housing): 1.5 mm

Inner flight bore matches shaft (16 mm)

Helix angle (mean diameter 38 mm): 26.7°

D — Shaft and Mounting

Nominal shaft diameter: 16 mm

Left shaft extension: 40 mm

Right coupling zone: 80 mm

Bearing bore: 16 mm

Bearing centers: Left 20 mm, Right 310 mm (span = 290 mm)

Coupling flange OD: 80 mm (4 × M8 holes)

Keyway width: 4 mm, depth: 1.3 mm

E — Tolerances and Surface Finish

Outer diameter tolerance: ±0.2 mm

Shaft tolerance: +0.00 / -0.02 mm

Flight thickness tolerance: ±0.2 mm

Axial pitch tolerance: ±0.5 mm per pitch

Surface finish (contact area): Ra ≤ 1.6 µm

F — Coordinate Reference (X-axis)

X = 0 mm: Left shaft end (hopper side)

X = 20 mm: Left bearing center

X = 40 mm: Start of flighted zone

X = 40–300 mm: Flighted zone (260 mm total)

X = 310 mm: Right bearing center

X =350mm~420mm:Shaft/coupling end

Note for CAD Design: Model the auger as a single-start helical flight with parameters: Outer Diameter =

60 mm, Shaft Diameter = 16 mm, Pitch = 60 mm, Flight Thickness = 4 mm, Clearance = 1.5 mm, Total

Length = 350 mm (including coupling and bearings). Use SS316 material and include bearing seats and

motor coupling zones per the specified coordinates.