CHAPTER SIX

RESULTS DISCUSSION AND EVALUATION

────────────────────

Performance Evaluation, Industrial Integration,

Mechatronic Synergy, and Engineering Contribution

# 6.1 Evaluation of Machine Performance

This section synthesizes the analytical and simulation results to evaluate the overall performance of the designed VFFS auger filling system. Performance is assessed against the target specifications established in Chapter 3, with particular attention to dosing accuracy, throughput capability, and structural reliability.

## 6.1.1 Precision under Variable Operating Speeds

The dosing accuracy of volumetric auger fillers is influenced by rotational speed through several mechanisms: centrifugal effects on material distribution, slip between particles and flight surfaces, and backflow through radial clearance. The designed system addresses these factors through optimized auger geometry and precise servo control.

The analytical model predicts volumetric efficiency of 65% at the nominal operating speed of 135 RPM. This efficiency factor accounts for:

• Particle slip at flight surfaces (estimated 15% reduction)

• Backflow through radial clearance (estimated 10% reduction)

• Incomplete flight filling at auger inlet (estimated 10% reduction)

The servo motor's 23-bit absolute encoder provides positioning resolution of 4.29 × 10⁻⁵ degrees per count, enabling precise angular control for volumetric dosing. At the target dose of 500g requiring 3.95 revolutions, the positioning error contribution to weight variation is negligible (<0.01%).

The primary source of dosing variation is bulk density fluctuation. For granular salt, bulk density can vary by ±5-10% depending on consolidation, moisture content, and particle packing. The control system implements compensation algorithms that adjust the target revolution count based on real-time density estimation from previous cycle weight feedback.

The predicted dosing accuracy is ±1.2% under normal operating conditions, satisfying the ±1.5% requirement with margin for process variation.

## 6.1.2 Structural Reliability under Peak Loads

The structural analysis confirms that all components operate with substantial safety margins under worst-case loading conditions. The FEA results presented in Chapter 5 demonstrate:

• Maximum von Mises stress: 32.8 MPa (16% of yield strength)

• Factor of safety against yield: 6.25

• Maximum deflection: 0.0021 mm (0.14% of radial clearance)

• Operating speed / Critical speed: 3.4% (exceptional resonance margin)

The safety factor of 6.25 exceeds typical industrial requirements (2.0-3.0) for rotating machinery, providing confidence in long-term reliability. The conservative design accommodates unexpected overload conditions such as material caking, startup with compacted product, or foreign object intrusion.

The thermal analysis confirms that thermal effects are negligible for dosing accuracy. The maximum auger temperature of 42°C causes thermal expansion of only 0.076 mm, representing 5% of the radial clearance. The design provides adequate margin for thermal expansion under all operational scenarios.

[Insert Figure Here]

Figure 6.1: Performance summary chart showing dosing accuracy, structural safety factor, and thermal margin against target specifications.

# 6.2 Suitability for VFFS Industrial Integration

The designed auger filling system is evaluated for integration with industrial VFFS packaging machines. Key integration considerations include mechanical interfaces, control system compatibility, and operational workflow.

## 6.2.1 Mechanical Integration

The compact footprint of the auger filler assembly (approximately 400mm × 300mm × 600mm) is compatible with typical VFFS machine configurations. The vertical discharge orientation aligns with standard bag forming tubes, and the tri-clamp hopper connection facilitates quick changeover for cleaning or product switching.

The support frame is designed to accommodate load cell integration for future gravimetric feedback implementation. Mounting provisions for weigh system components are included without compromising structural integrity.

## 6.2.2 Control System Integration

The ESP32-based control system provides multiple communication interfaces for VFFS integration:

• Digital I/O: Start/stop signals, cycle complete indication, fault status

• Analog input: Speed command from VFFS master controller

• Serial communication: RS-485 Modbus RTU for parameter exchange

• Ethernet: Optional TCP/IP for Industry 4.0 connectivity

The control system implements the standard VFFS timing sequence: bag presence confirmation, auger filling cycle, cycle complete signal, and interlock for sealing operation. Integration with existing VFFS controllers requires only digital I/O connections, minimizing installation complexity.

## 6.2.3 Operational Workflow

The designed system supports continuous operation at 15 PPM with the following workflow:

1.VFFS machine forms bag and positions under filling tube

2.Bag presence sensor confirms correct positioning

3.VFFS controller signals auger filler to start filling cycle

4.Auger rotates through calculated angle (3.95 rev) with S-curve profile

5.Spinner plate stops material flow at cycle completion

6.Cycle complete signal returned to VFFS controller

7.VFFS advances film and seals filled bag

The 2.0 second filling time provides margin within the 4.0 second total cycle, allowing for film transport and sealing operations without throughput limitation.

# 6.3 Discussion of Mechatronic Synergy (How Control Impacts Mechanical Life)

The integration of advanced servo control with mechanical design creates synergistic benefits that extend beyond the capabilities of either domain in isolation. This section discusses how control system design impacts mechanical component life and overall system reliability.

## 6.3.1 Motion Profile Optimization

The S-curve motion profile implemented in the servo drive provides gradual acceleration and deceleration, reducing mechanical shock and vibration compared to trapezoidal profiles. The benefits include:

• Reduced peak torque demand (approximately 20% lower than trapezoidal)

• Minimized mechanical resonance excitation

• Extended bearing life through reduced impact loading

• Improved dosing consistency through smooth material flow

The jerk-limited profile (maximum 5000 RPM/s²) ensures that acceleration changes occur gradually, preventing mechanical shock that could cause premature wear or fatigue failure.

## 6.3.2 Closed-Loop Position Control

The closed-loop position control with 23-bit encoder feedback provides real-time error correction that compensates for mechanical imperfections and load variations. Benefits include:

• Consistent dosing accuracy regardless of load variation

• Automatic compensation for mechanical backlash

• Early detection of mechanical anomalies through position error monitoring

• Elimination of homing requirements with absolute encoder

## 6.3.3 Condition Monitoring and Predictive Maintenance

The servo drive's built-in monitoring capabilities enable condition-based maintenance strategies that extend component life:

• Motor current monitoring detects bearing degradation or material obstruction

• Position error trending identifies mechanical wear before failure

• Temperature monitoring prevents thermal overload conditions

• Vibration analysis (if implemented) detects imbalance or misalignment

By transitioning from time-based to condition-based maintenance, the system achieves higher availability and lower life-cycle cost compared to mechanically-controlled alternatives.

[Insert Figure Here]

Figure 6.2: Mechatronic synergy diagram showing interaction between mechanical design, servo control, and operational benefits.

# 6.4 Contribution to Engineering Knowledge

This graduation project contributes to engineering knowledge through the development and validation of an integrated design methodology for VFFS auger filling systems. The specific contributions include:

## 6.4.1 Integrated Design Methodology

The project demonstrates a systematic approach combining analytical modeling, numerical simulation, and design validation within a unified framework. This methodology can be applied to similar mechatronic system development, providing a template for systematic optimization of mechanical and control system integration.

## 6.4.2 Auger Geometry Optimization for Salt Packaging

The specific auger geometry (60mm diameter, 16mm shaft, 60mm pitch) optimized for 500g salt packaging represents a documented design point that can inform future applications. The correlation between analytical predictions and FEA results validates the design approach and provides confidence for scaling to other package sizes.

## 6.4.3 Thermal-Structural Coupling Analysis

The thermal-structural coupling analysis demonstrates that thermal effects from adjacent sealing operations are negligible for auger dosing accuracy when proper thermal isolation is maintained. This finding informs design decisions for VFFS integration and clarifies the thermal design margin requirements.

## 6.4.4 Servo Control Implementation for Powder Dosing

The implementation of advanced servo control (S-curve profiles, absolute positioning, real-time feedback) for powder dosing applications demonstrates performance improvements achievable through mechatronic integration. The documented control architecture provides a reference for similar applications.

Table 6.1 summarizes the key performance achievements relative to target specifications:

| Parameter | Target | Achieved | Status |
| --- | --- | --- | --- |
| Dosing Accuracy | ±1.5% | ±1.2% | Exceeded |
| Production Rate | 15 PPM | 15 PPM | Met |
| Filling Time | <2.0 s | 1.76 s | Exceeded |
| Safety Factor | >2.0 | 6.25 | Exceeded |
| Deflection | <1.5 mm | 0.0021 mm | Exceeded |
| Frequency Ratio | >1.4 | 29.2 | Exceeded |
| Operating Temp | <60°C | 42°C | Exceeded |

Table 6.1: Performance Summary vs. Target Specifications

The designed system meets or exceeds all target specifications, with particularly strong performance in structural reliability (FoS = 6.25) and resonance margin (29.2× operating speed). The predicted dosing accuracy of ±1.2% satisfies the ±1.5% requirement with margin for process variation.

# References

[1] Ulrich, K. T., & Eppinger, S. D. (2015). Product Design and Development (6th ed.). McGraw-Hill Education.

[2] Panasonic Industry. (2022). MINAS A6 Series AC Servo Motor Technical Reference. Panasonic Industry Europe GmbH.

[3] El Kassem, B. (2021). Machine Design for Auger Dosing of Powders by utilizing DEM Simulations. Ph.D. dissertation, RWTH Aachen University.

[4] Hohmann, H. J. (1988). Filling characteristics and dosing accuracy of volumetric dosing devices. Packaging Technology and Science, 1(3), 123-135.

[5] CEMA. (2015). Screw Conveyors for Bulk Materials. ANSI/CEMA Standard 350-2015.