تقرير المرحلة الأولى
تصميم لولب التعبئة (Auger Screw) ضمن آلة VFFS — Phase 1
(وثائق مرجعية ومراجع متكاملة)

# الملخص التنفيذي — Executive Summary

هذا التقرير يغطي المرحلة الأولى من تصميم لولب التعبئة (Auger) لآلة تعبئة عمودية VFFS، مع التركيز على: المعادلات الهندسية المستخدمة، القِيَم المُعتمدة، الحسابات الرقمية خطوةً بخطوة لنطاق الأوزان (100–500 g)، وكذا المصادر والأوراق العلمية التي استُنِدَت إليها مع روابط مباشرة أمام كل معادلة أو قسم.

This report is Phase 1 documentation for the auger screw design for a VFFS powder filler. It contains equations, input values, detailed numeric calculations for bag weights 100–500 g, and references with direct links.

## 1. المعطيات والافتراضات (Given data & assumptions)

| البند (Item) | الرمز (Symbol) | القيمة (Value) | ملاحظة / مرجع (Note / Source URL) |
| --- | --- | --- | --- |
| كثافة bulk (استعمالي) | ρ | 1300 kg/m³ | قيمة زودها المستخدم؛ مرجع كثافة الأمثلة: https://kg-m3.com/material/salt-fine-table |
| معامل الملء (Volumetric fill factor) | ηv | 0.60 (افتراضي للعملية) | قيمة افتراضية؛ مرجع بحثي: RWTH thesis (DEM) https://publications.rwth-aachen.de/record/814145/files/814145.pdf |
| قطر اللولب الخارجي | D | 60 mm | قيمة تصميمية مُعطاة |
| قطر العمود المركزي (Shaft) | D_s | 16 mm | قيمة مُعطاة؛ ممكن ترقية إلى 20 mm |
| الخطوة (Pitch) | p | 60 mm | اعتماد شائع (p = D)؛ مرجع: KWS https://www.kwsmfg.com/engineering-guides/screw-conveyor/screw-conveyor-capacity/ |
| عدد اللفات | N | 4 لفات | تصميم single-start helix |
| زمن التعبئة | t_fill | 2 s | زمن التعبئة المعيّن من المستخدم |
| نطاق الوزن للكيس | m_bag | 100–500 g | نطاق الحسابات المطلوبة |

## 2. المعادلات الأساسية (Equations) — مع مرجع لكل معادلة

V_t = (π / 4) · (D² - D_s²) · p

Volume per revolution — used in screw conveyor capacity calculations. See KWS engineering guide and Conveyor Engineering manual.

مرجع: https://www.kwsmfg.com/engineering-guides/screw-conveyor/screw-conveyor-capacity/ , https://www.screwconveyorparts.com/wp-content/uploads/2018/06/Conveyor-Engineering-Screw-Conveyor-Manual-2.20-SCP.pdf

m_rev = ρ · ηv · V_t

Mass per revolution = density × volumetric efficiency × volume per revolution. See All-Fill volumetric brochure and RWTH thesis.

مرجع: https://allfill.co.uk/wp-content/uploads/2019/06/Series-10-Volumetric-Brochure.pdf

, https://publications.rwth-aachen.de/record/814145/files/814145.pdf

n = m_bag / m_rev

Revolutions required per bag. Standard volumetric filler control approach (All-Fill).

مرجع: https://all-fill.com/equipment/auger-fillers/ , https://allfill.co.uk/wp-content/uploads/2019/06/Series-10-Volumetric-Brochure.pdf

RPM = 60 · n / t_fill

Convert revs/sec to revs/min for motor selection and control.

مرجع: Basic conversion; applied in All-Fill brochure.

α = atan(p / (π · D_m)) where D_m = (D + D_s)/2

Helix angle formula (mean diameter) used for geometry and helix layout; see RWTH thesis and screw feeder literature.

مرجع: https://publications.rwth-aachen.de/record/814145/files/814145.pdf , https://www.researchgate.net/publication/268032034_Design_and_sizing_of_screw_feeders

## 3. الحسابات الرقمية خطوة بخطوة (100–500 g)

حساب V_t (الحجم لكل دورة): V_t = (π/4)(D² − D_s²)p = 0.00015758 m³/rev (157.58 cm³/rev)

حساب m_rev (الكتلة لكل دورة): m_rev = ρ·ηv·V_t = 0.122914 kg/rev (122.91 g/rev)

| m_bag (g) | m_bag (kg) | m_rev (g/rev) | revs per bag (n) | revs/sec (n/t_fill) | RPM (60*n/t_fill) |
| --- | --- | --- | --- | --- | --- |
| 100 | 0.1000 | 122.91 | 0.814 | 0.407 | 24.4 |
| 150 | 0.1500 | 122.91 | 1.220 | 0.610 | 36.6 |
| 200 | 0.2000 | 122.91 | 1.627 | 0.814 | 48.8 |
| 250 | 0.2500 | 122.91 | 2.034 | 1.017 | 61.0 |
| 300 | 0.3000 | 122.91 | 2.441 | 1.220 | 73.2 |
| 400 | 0.4000 | 122.91 | 3.254 | 1.627 | 97.6 |
| 500 | 0.5000 | 122.91 | 4.068 | 2.034 | 122.0 |

## 4. حساسية النتائج (تغير ηv وρ)

النطاقات العملية لاختبار الحساسية: ηv∈{0.4,0.5,0.6} و ρ∈{1200,1300,1380} kg/m³

| ηv | ρ (kg/m³) | m_rev (g/rev) | revs for 250 g | RPM for 250 g (t_fill=2s) |
| --- | --- | --- | --- | --- |
| 0.40 | 1200 | 75.64 | 3.305 | 99.2 |
| 0.40 | 1300 | 81.94 | 3.051 | 91.5 |
| 0.40 | 1380 | 86.99 | 2.874 | 86.2 |
| 0.50 | 1200 | 94.55 | 2.644 | 79.3 |
| 0.50 | 1300 | 102.43 | 2.441 | 73.2 |
| 0.50 | 1380 | 108.73 | 2.299 | 69.0 |
| 0.60 | 1200 | 113.46 | 2.203 | 66.1 |
| 0.60 | 1300 | 122.91 | 2.034 | 61.0 |
| 0.60 | 1380 | 130.48 | 1.916 | 57.5 |

## 5. المراجع (References) — روابط مباشرة للمصادر المستخدمة

Screw Conveyor Capacity — KWS Engineering Guide: https://www.kwsmfg.com/engineering-guides/screw-conveyor/screw-conveyor-capacity/

Design and sizing of screw feeders — ResearchGate (Bortolamasi & Fottner): https://www.researchgate.net/publication/268032034_Design_and_sizing_of_screw_feeders

Machine Design for Auger Dosing of Powders — RWTH Aachen (thesis, PDF): https://publications.rwth-aachen.de/record/814145/files/814145.pdf

All-Fill Auger Fillers (product & technical): https://www.all-fill.com/equipment/auger-fillers/

Screw Conveyor Design and Component Manual (Conveyor Engineering): https://www.screwconveyorparts.com/wp-content/uploads/2018/06/Conveyor-Engineering-Screw-Conveyor-Manual-2.20-SCP.pdf

Salt, fine table density — kg-m3: https://kg-m3.com/material/salt-fine-table

A Review on Screw Conveyors for Bulk Materials — ResearchGate (review): https://www.researchgate.net/profile/Supattarachai-Sudsawat-2/publication/361655904_A_Review_on_Screw_Conveyors_for_Bulk_Materials_in_Various_Applications/links/62be6ad37d27ac698c2a4d5a/A-Review-on-Screw-Conveyors-for-Bulk-Materials-in-Various-Applications.pdf

All-Fill Series 10 Volumetric Brochure (PDF): https://allfill.co.uk/wp-content/uploads/2019/06/Series-10-Volumetric-Brochure.pdf

Modeling the Discharge Rate of a Screw Conveyor (MDPI Agriculture): https://www.mdpi.com/2077-0472/14/7/1203

Auger filler manual (technical PDF): https://img1.wsimg.com/blobby/go/f36019eb-f077-446f-b5b6-39b8eacedf97/downloads/82417723235.pdf

Prepared by: Hassan M. (Project team)

Date: