CHAPTER THREE

# SYSTEM DESIGN AND SPECIFICATIONS

# SYSTEM DESIGN AND SPECIFICATIONS

## 3.1 Research Design Framework

The design of the Vertical Form-Fill-Seal (VFFS) auger filling system follows a systematic mechatronic engineering methodology that integrates requirements analysis, conceptual design, detailed engineering, and comprehensive validation through simulation. This structured approach ensures comprehensive coverage of all technical aspects—spanning mechanical structures, electronics, and software logic—while maintaining traceability from initial specifications to final implementation. The research design framework incorporates elements from established product development methodologies including Ulrich and Eppinger's Product Design and Development process, the V-model systems engineering approach [1], and the Model-Based Design (MBD) paradigm for control system synthesis.

The design process is organized into four sequential and concurrently engineered phases:

- Requirements Definition: Establishment of functional requirements, performance specifications, and constraints based on application analysis, material properties (e.g., salt ), and stakeholder needs.

- Conceptual Design: Development of system architecture, functional decomposition, kinematic planning, and selection of fundamental design concepts for both mechanical actuators and embedded control hardware.

- Detailed Design: Specification of component geometries, material selections, manufacturing processes, and the formulation of the distributed control architecture (incorporating dual-core processing, RTOS, and precise algorithm design).

- Design Validation: Verification of design adequacy through analytical modeling, structural numerical simulation, thermal system identification, and correlation analysis.

This chapter addresses Phases1,2 and 3, presenting the conceptual development and detailed specifications of the VFFS auger filling system. Chapter 4 presents the analytical modeling (Phase 4 validation methodology), while Chapter 5 documents the numerical simulation and correlation analysis.

Figure 3. 1:Design methodology flowchart showing the sequential phases from requirements definition through conceptual design, detailed design, and validation

## 3.2 Design Requirements and Constraints

Design requirements derive from the target application (packaging of 500g granular salt), industrial standards for food packaging machinery, and operational constraints imposed by the VFFS integration context. Requirements are categorized as functional (what the system must do) and non-functional (how the system must perform) [2].

### 3.2.1 Target Output (PPM: Packs Per Minute) and Accuracy (±σ)

The production throughput requirement establishes fundamental constraints on the mechanical and control system design. For the target application of 500g salt packaging, the following performance specifications are established:

• Target Production Rate: 20 bags per minute (20 PPM)

• Package Weight: 500 grams net content

• Weight Accuracy: ±1.5% .

• Filling Time: Maximum 0.8 seconds per cycle

• Cycle Time: 3.0 seconds total (including forming and sealing)

The accuracy specification of ±1.5% aligns with OIML R87 international standards for prepackaged goods and represents typical industrial requirements for medium-value food products. Achieving this accuracy with volumetric dosing requires careful control of bulk density variation and precise angular positioning of the auger screw [3].

The filling time constraint of 0.8 seconds per cycle establishes the required auger rotational speed. Based on volumetric displacement calculations (detailed in Chapter 4), delivering 500g of salt requires approximately 4 revolutions of the auger. The required rotational speed is therefore:

N = (4 rev / 0.8 s) × 60 s/min = 300 RPM (nominal)

A design speed of 300 RPM is selected to provide margin for operational variation and to accommodate potential efficiency losses under adverse conditions.

### 3.2.2 Sanitary and Material Standards (e.g., AISI 304 Stainless Steel)

As a food packaging machine, all product-contact components must comply with applicable food safety regulations and hygienic design principles. Key requirements include:

Material Requirements:

• All product-contact surfaces: Food-grade stainless steel (AISI 304 minimum, AISI 316L preferred for salt applications)

• Surface finish: Ra < 0.8 μm for product-contact areas

• No toxic materials: Compliance with FDA 21 CFR and EU Regulation 1935/2004

Hygienic Design Requirements:

• Smooth, continuous welds with no crevices for bacterial harborage

• Self-draining surfaces with no standing liquid zones

• Quick-disconnect components for cleaning access

• No dead zones where product can accumulate and degrade

• Compatibility with Clean-Out-of-Place (COP) procedures

Choosing AISI 316L stainless steel for all salt contact components provides superior chloride corrosion resistance compared to AISI 304. The molybdenum content (2-3%) in 316L forms protective oxide layers that resist pitting in saline environments, but due to the unavailability of AISI 316L stainless steel in Yemen, AISI 304 stainless steel was chosen, which also meets health requirements and food safety regulations [4].

Figure 3. 2:Hygienic design principles for food-contact components, showing smooth transitions, self-draining surfaces, and cleanable geometry

## 3.3 Conceptual Development

Conceptual development transforms abstract requirements into concrete system architecture through functional decomposition and concept selection. This section presents the functional breakdown of the VFFS auger machine and the rationale for key design decisions.

### 3.3.1 Functional Decomposition (Dosing System, Packaging & Sealing System.)

- Dosing System Architecture :

The Dosing system is decomposed into four primary functional subsystems, each addressing specific aspects of the material handling and dosing process:

- Hopper Subsystem:

The hopper provides temporary storage of bulk material and ensures consistent feeding of the auger. Key design considerations include:

• Capacity:  20 L=16 kg for salt.

• Dimensions: Total Height (H) 330 mm , Top Diameter (Dtop) 446 mm, Outlet Diameter (Dout) 65 mm, Wall Angle (from horizontal)  60° .

• Geometry: Conical bottom with wall angle exceeding the angle of repose to ensure mass flow

• Material: AISI 304 stainless steel with electropolished interior surface

The hopper design incorporates a steep wall angle of 60° relative to the horizontal, which exceeds both the angle of repose (30-35°) and the wall friction angle (15-25°) of salt. This design ensures mass flow rather than funnel flow, preventing stagnation zones and maintaining consistent bulk density at the auger inlet.

- Agitator System and Coaxial Drive Architecture Design:

- Agitator Subsystem:

The agitator prevents bridging and ratholing in the hopper while promoting consistent bulk density at the auger inlet. Design features include:

- Number of Blades: 2 (diametrically opposed at 180°)

- Dimensions: Upper Blade Position: 148.5 mm from top (45% of cone height), Lower Blade Position: 297 mm from top (90% of cone height), Blade Width: 60 mm , Blade Thickness: 8 mm

•     Configuration: Counter-rotating paddle design sweeping the hopper periphery

•     Speed: 30 RPM (independent drive from auger)

•     Clearance: 3-5 mm from hopper walls to prevent interference

•      Drive: DC motor .

The agitator performs three critical functions:

1. Bridge Breaking: Continuously destroys cohesive arches forming above the screw

2. Homogenization: Re-mixes fine and coarse particles to minimize segregation and stabilize bulk density

3. Force Feeding: Actively pushes powder toward the auger flights to ensure full screw filling in each revolution

- Agitator System and Coaxial Drive Architecture Design:

To ensure continuous mass flow and prevent bridging, the internal agitation system operates independently from the main auger screw. The design utilizes a highly specialized coaxial drive architecture that allows the central auger shaft (Blender Auger Long) to rotate at 300 RPM (CW) while the outer Blender Seat, which holds the Agitator / Blender blades, rotates simultaneously at 30 RPM (CCW).

This kinematic decoupling is achieved through a precision-engineered concentric assembly:

- Inner Transmission: The primary high-speed torque is transmitted through the Auger Shaft and the Blender Auger Long directly to the Auger Screw.

- Outer Transmission: The low-speed counter-rotation is driven by an independent motor housed in the Blender Motor Case. The torque is transmitted via a timing belt system utilizing an HTPA50S5M150-A-H40 driven pulley and a C-HTPA20S5M150-A-P15 drive pulley, which are connected to the Blender Seat

- Bearing Architecture and Mechanical Decoupling:

The fundamental challenge in this coaxial design is physically isolating the high-speed and low-speed rotational dynamics to prevent friction-induced torque transfer. This is resolved by an advanced bearing and sleeve configuration:

Inner Isolation (Shaft-to-Seat): Deep groove ball bearings (B6702ZZ and B6804ZZ) are press-fitted between the high-speed Blender Auger Long shaft and the intermediate Coupling Sleeve / Blender Seat. These bearings completely isolate the internal 300 RPM rotation from the external 30 RPM rotation.

Outer Support (Seat-to-Frame): The entire rotating agitator assembly is structurally supported by the Outer Bearing Seat, which is rigidly bolted to the Funnel Plate Seat and the Main Plate. Additional bearings (B6809ZZ and B6902ZZ) are placed between the Blender Seat and the Outer Bearing Seat, allowing the agitator to rotate smoothly relative to the stationary machine frame.

Axial Retention and Preload: To prevent any axial displacement or vibrations during the start-stop dosing cycles, precision locknuts (MUEN15 and MUEN45) are utilized. These locknuts secure the inner and outer bearing races, ensuring strict geometric alignment and preventing the tight radial clearance (1.5 mm) from deviating during operation.

Environmental Protection: All selected bearings feature double metal shields (ZZ designation). This is a critical design choice to prevent the highly abrasive and hygroscopic sodium chloride dust from penetrating the rolling elements, thereby extending the fatigue endurance of the mechanical assembly.

- Auger Subsystem:

The auger screw is the core dosing element, providing positive displacement metering of material. Key specifications include:

• Outer diameter: 60 mm

• Shaft diameter: 16 mm

• Pitch: 60 mm (square pitch configuration)

• Radial clearance: 1.5 mm maximum

• Material: AISI 304 stainless steel with electropolished flights

The auger geometry follows the square pitch configuration (pitch equal to outer diameter), which is optimal for free-flowing granular materials. This configuration provides stable flow with minimal particle segregation and predictable volumetric displacement.

- Discharge Nozzle and Spinner Plate:

The discharge mechanism controls material flow during and after the dosing cycle. The spinner plate provides flow cutoff when rotation stops, preventing dribble that would compromise weight accuracy. Design parameters include:

• Spinner plate diameter: 80 mm

• Clearance to funnel lip: 2-3 mm

• Operating speed: Synchronous with auger (300 RPM)

• Function: Centrifugal dispersion during rotation, angle-of-repose containment when stopped

The spinner plate operates on the principle of balance between centrifugal force and angle of repose. When the auger rotates, centrifugal force pushes salt particles radially outward through the gap between the plate and the funnel lip. Upon auger stop, centrifugal force vanishes, and the accumulated salt forms a "natural product dam" that prevents leakage without requiring complex mechanical valves.

Figure 3. 3: Functional decomposition diagram showing the four primary subsystems and their interactions

- Packaging and Sealing System Architecture:

The packaging subsystem transforms the planar thermoplastic film into a hermetically sealed pouch.

- Forming Tube Subsystem:

The forming tube constitutes the core component of the VFFS system, facilitating the optimal folding of the film.

• Dimensions: External diameter of 83 mm (yielding a 120 mm bag width + 10 mm seal), internal diameter of 77 mm, and an overall length of 400 mm.

• Forming Collar: Engineered with a 25° angle for optimal film folding fluidity.

• Manufacturing Details: Constructed from AISI 304 stainless steel with a highly polished surface ( Ra≤0.4 mμ) and a roundness tolerance of ±0.05 mm.

Figure 3.3: Forming tube and collar assembly, illustrating the 25° forming angle and the mounting flange

- Sealing Mechanisms (Horizontal & Vertical):

Thermal integrity is paramount for hygroscopic materials like NaCl.

• Horizontal Sealing Jaws: These 220 mm long jaws create the transverse seals and sever the     pouch. They apply 300–500 N of force and utilize a 300W cartridge heater. The 8 mm sealing surface is bifurcated into dual 3 mm lines with a central groove for a 0.5 mm cutting blade, featuring a cross-hatch pattern (0.2 mm depth) for optimal grip.

• Vertical Sealing Bars: A 400 mm bar operating at 140–160°C under 3 Bar of pneumatic pressure. It features a 10 mm sealing width and a 3 mm PTFE thermal insulation layer, adjustable horizontally by ±15 mm.

- Film Handling and Pull Mechanism:

This system extracts film from the supply roll with controlled velocity, precision, and modulated tension.

• Film Unwinder & EPC: Equipped with a mechanical brake for tension modulation, a dancer arm sensor for real-time feedback, and an Edge Position Control (EPC) photoelectric sensor driving a corrective servo/stepper actuator to maintain accurate film tracking.

• Pull Belt Assembly: Utilizes counter-rotating nip rollers (80 mm diameter, 300 mm width) coated in 60 Shore A durometer silicone rubber. Driven by a 450–750W servo motor with a 1:10 reduction ratio, ensuring precise kinematic synchronization.

### 3.3.2 Selection of Actuation (Servo-driven vs. Induction)

The selection of motor technology for auger drive represents a critical design decision affecting precision, cost, and system complexity. Three primary options were evaluated: induction motor with clutch-brake, stepper motor, and servo motor [5].

Option 1: Induction Motor with Clutch-Brake

Traditional auger fillers employed induction motors running continuously with mechanical clutch-brake mechanisms to control auger rotation. While mechanically simple and cost-effective, this approach suffers from significant limitations:

• Poor positioning accuracy due to clutch engagement variability

• Mechanical wear requiring frequent maintenance

• Limited dynamic response (slow acceleration/deceleration)

• Inability to implement advanced motion profiles

Option 2: Stepper Motor

Stepper motors provide open-loop position control without feedback devices. While offering improved positioning compared to clutch-brake systems, stepper motors exhibit several disadvantages:

• Risk of missed steps under load, causing cumulative positioning errors

• Limited torque at high speeds

• Audible noise and vibration at certain speeds

• No verification of actual position achieved

Option 3: Servo Motor (Selected)

Servo motor systems combine high-performance brushless motors with closed-loop feedback control, offering superior performance for precision dosing applications. The selected Panasonic MINAS A6 AC servo motor provides:

• 23-bit absolute encoder (8,388,608 pulses per revolution)

• High dynamic response (acceleration to rated speed in <50 ms)

• Full torque available across entire speed range

• Electronic cam and motion profile capability

• Real-time position verification and error correction

The servo motor is coupled to a 20:1 planetary gearbox, providing output speed of 300 RPM at the auger while multiplying motor torque by the gear ratio (minus efficiency losses). The planetary configuration offers high torque density, low backlash (<3 arcminutes), and efficient power transmission [6].

The selection of servo motor technology is justified by the following quantitative analysis:

Positioning Resolution:

The 23-bit encoder provides angular resolution of:

θ_res = 360° / 2^23 = 360° / 8,388,608 = 4.29 × 10^-5 degrees per count

For a 500g dose requiring 4 revolutions, the positioning error contribution to weight variation is:

Error = (4.29 × 10^-5°) × (4 rev) × (500g / 360°) = 2.38 × 10^-4 g

This negligible error confirms that servo positioning accuracy exceeds the requirements for ±1.5% dosing accuracy.

Figure 3. 4Servo motor and planetary gearbox assembly showing encoder feedback, drive coupling, and mounting configuration

## 3.4 Final Mechanical Architecture (CAD)

The final mechanical design integrates all functional subsystems into a single, optimized unit that is easier to manufacture and maintain. This section presents the machine's mechanical design in the SolidWorks software.

### 3.4.1 Dosing System CAD:

Figure:3.5 Dosing System CAD

- Final Hopper Dimensions:

Table 3. 1:Final Hopper Design Dimensions

| Parameter | Value |
| --- | --- |
| Total Height (H) | 330 mm |
| Top Diameter (Dtop) | 446 mm |
| Outlet Diameter (Dout) | 65 mm |
| Wall Angle (from horizontal) | 60° |
| Wall Angle (from vertical) | 30° |
| Capacity | 20 L |
| Material | AISI 304 stainless steel |

Figure:3.6 Hopper CAD

- Final Blade Configuration

- Number of Blades: 2 (diametrically opposed at 180°)

- Blade Type: Skewed paddle with 25° rake angle

- Blade Width: 60 mm

- Blade Thickness: 8 mm

- Upper Blade Position: 148.5 mm from top (45% of hopper height)

- Lower Blade Position: 297 mm from top (90% of hopper height)

- • Material: AISI 304 stainless steel

Figure:3.7Long Blade CAD

Figure:3.8 Short Blade CAD

- Final Agitator System and Coaxial Drive Architecture Design:

Figure:3.9 Agitator System CAD

- Final Auger Screw Design:

The auger screw and surrounding funnel assembly constitute the core dosing mechanism. The design employs a straight (non-tapered) auger with square pitch geometry optimized for free-flowing granular materials.

Auger Screw Specifications:

• Outer diameter (D): 60 mm

• Shaft diameter (Ds): 16 mm

• Pitch (P): 60 mm

• Flight thickness: 3 mm

• Number of effective pitches: 4.5

• Overall length: 320 mm

• Material: AISI 304 stainless steel

Figure:3.10 Auger CAD

- Final Spinner plate :

• diameter: 80 mm

Figure:3.11Spinner Plate& Auger CAD

- Funnel Assembly Specifications:

• Inner diameter: 63 mm (providing 1.5 mm radial clearance)

• Length: 150 mm

• Material: AISI 304 stainless steel

The radial clearance of 1.5 mm between auger flight tip and funnel wall represents a compromise between dosing accuracy and manufacturing tolerance. Smaller clearance reduces backflow but increases risk of metal-to-metal contact due to thermal expansion or mechanical deflection. The 1.5 mm specification provides adequate margin while maintaining acceptable volumetric efficiency [7].

The clearance specification is validated through thermal expansion analysis:

ΔL = α × L × ΔT

Where α = 16.0 × 10^-6 /°C (thermal expansion coefficient for AISI 304L)

L = 280 mm (auger length)

ΔT = 42 - 25 = 17°C (maximum temperature rise)

ΔL = 16.0 × 10^-6 × 280 × 17 = 0.076 mm

The thermal expansion of 0.076 mm represents only 5% of the radial clearance, confirming adequate margin for thermal effects.

Figure 3.12 CAD model of  funnel assembly

### 3.4.2 Packaging and Sealing System CAD:

Figure:3.13Packaging and Sealing System CAD

- Final Forming Tube Design :

- External diameter: 83 mm (for producing 120 mm width bags + 10 mm seal)

- Internal diameter: 77 mm (wall thickness 3 mm)

- Overall length: 400 mm

- Forming zone length: 300 mm

- Collar angle: 25° for optimal film folding fluidity

- SolidWorks Design Procedure:

1. Primary Sketch: External circle Ø83 mm, Internal circle Ø78 mm, Extrude 400 mm

2. Detail Addition: Conical collar at 25° (Revolve Feature), Mounting holes 4×M8, Upper flange 10 mm

3. Finishing: Fillet radius 2 mm, Mirror Polish, Apply material SS 304L

Figure:3.14 Forming Tube CAD

- Final Horizontal Sealing Jaws Design:

  - Jaw length: 220 mm

  - Sealing surface width: 8 mm (bifurcated into dual 3 mm sealing lines with 2 mm central groove)

  - Stroke distance: 25 mm

  - Sealing force: 300–500 N

  - Heating system: 300W cartridge heating element

Figure:3.15 Horizontal Sealing Jaws CAD

- Final Vertical Sealing Bars Design:

- Bar length: 400 mm

- Sealing surface width: 10 mm

- Pressure range: 3 Bar

- Operating temperature: 140–160°C

Figure:3.16 Vertical Sealing Bars  CAD

- Final Film Pull Mechanism Design:

  - Mechanism type: Counter-rotating nip rollers

  - Roller diameter: 80 mm

  - Roller width: 300 mm

  - Motor: 450–750W servo motor

  - Reduction ratio: 1:10

Figure:3.17 Final Film Pull Mechanism

### 3.4.2 Drive Train and Power Transmission

The drive train transmits power from the servo motor to the auger shaft while providing speed reduction and torque multiplication. The selected configuration comprises:

Prime Mover: Panasonic MINAS A6 AC Servo Motor

• Model: MSMF042L1U

• Rated power: 400W

• Rated torque: 1.27 N·m

• Maximum torque: 3.82 N·m

• Rated speed: 3000 RPM

• Max. rotational speed : 6000 RPM

• Encoder resolution: 23-bit absolute (8,388,608 pulses/rev)

Gearbox: Neugart PLE Planetary Gearbox

• Model: PLE 060

• Reduction ratio: 20:1

• Output torque (nominal): 25 N·m

• Output torque (maximum): 40 N·m

• Backlash: <3 arcminutes

• Efficiency: >97%

Power Transmission Calculations:

The effective output torque at the auger shaft is calculated as:

T_out = T_motor × i × η = 1.27 N·m × 20 × 0.97 = 24.6 N·m

Where i is the gear ratio and η is the gearbox efficiency. This output torque provides substantial margin above the calculated requirement of approximately 18-20 N·m under worst-case conditions, ensuring reliable startup even with compacted material [8].

The power transmission efficiency is validated through energy balance:

P_in = 400W (motor rated power)

P_out = T_out × ω = 24.6 N·m × (2π × 135/60) rad/s = 347 W

η_system = P_out / P_in = 347 / 400 = 86.8%

The system efficiency of 86.8% accounts for gearbox losses (3%), bearing friction (5%), and auger mechanical losses (5%).

## 3.5 Mechatronic System Architecture

## The core of the control system lies in transforming the mechanical structure of the VFFS machine into an intelligent, deterministic system capable of synchronizing mechanical and thermal operations at high speeds with absolute reliability [14]. To ensure consistent seal quality during continuous operation, the thermal control subsystem was designed to maintain the sealing jaws at 150°C with a high precision of ±1°C [18]. Because the sealing process is heavily influenced by thermal inertia, heat losses, and continuous environmental fluctuations, a Model-Based Design (MBD) methodology was adopted to tune system performance, superseding traditional empirical trial-and-error methods.

### 3.5.1 Distributed Processing Strategy (Master-Slave Topology)

## To mitigate the computational bottlenecks arising from simultaneous graphical rendering and thermo-kinetic control, a distributed control architecture was engineered utilizing two ESP32 microcontrollers [16]:

## Master Node (Machine Controller): Acts as the "executive brain" of the machine. It is dedicated to executing the thermal PID algorithm, polling sensors, and generating motor pulses. This isolation guarantees that the machine's stability remains immune to any latency introduced by the user interface, fulfilling the requirements for rapid response and the elimination of steady-state error [14, 15].

## Slave Node (HMI Controller): Functions as a dedicated graphical processor for the Human-Machine Interface (HMI), maintaining communication with the Master Node via a high-speed UART protocol [16, 20].

Figure 3.18 General control system architecture block diagram showing the Master ESP32, Slave ESP32, sensors, SSR, and UART communication.

### 3.5.2 Operating System and Task Allocation (FreeRTOS & Dual-Core Strategy)

## To achieve maximum stability, the dual-core processor of the ESP32 was leveraged via the FreeRTOS operating system to facilitate true multitasking [16, 17]. The core concept hinges on isolating precise hardware operations from application logic delays. Tasks were allocated according to strict priorities as follows:

## Core 0 (Hardware & Time-Critical Tasks):

## Fault Handler (Priority 5): Provides an instant response to Emergency Stop (E-STOP) interrupts, bypassing all software delays [17].

## Motion Controller (Priority 4): Manages precise stepper motor control and pulse generation via the hardware RMT peripheral [16].

## Thermal PID Controller (Priority 3): Executes the PID algorithm with a precise sample time of 1 second, an optimal duration for systems exhibiting significant thermal inertia [15, 19]. The controller's output is not utilized as a direct analog signal; instead, it is converted into Time-Proportional Control to drive the Solid State Relay (SSR) [15].

## Core 1 (Application Logic & UI):

## Control Loop (Priority 2): Manages the sequential logic of the machine via a Finite State Machine (FSM), including operational sequences and execution delays [17].

## HMI (Priority 1): Handles button inputs, updates system status, and communicates with the Slave Node [20].

### 3.5.3 Mathematical Modeling and Thermal PID Controller

## The thermal control system of the sealing jaws is one of the most critical subsystems required to guarantee hermetic bag seals. A Model-Based Design (MBD) methodology was employed through the following procedural steps:

- System Identification:

Based on the physical properties of the stainless-steel jaws (approximated as a 1 kgthermal mass) and accounting for conductive and convective heat losses, the system was dynamically modeled within the MATLAB/Simscape environment. By subjecting the open-loop model to a step response test, a first-order plus dead time (FOPDT) transfer function was derived:

G(s) = \frac{218.2}{272.7s + 1} e^{-2s}

- PID Controller Tuning:

Utilizing the MATLAB PID Tuner application, optimal parameters for a discrete PID controller were established. These parameters ensure thermal stability at the 150°Csetpoint, characterized by a maximum overshoot of less than $1\%$ and a steady-state error tolerance of  1°C:

- Proportional (P): 1.0

- Integral (I): 0.004

- Derivative (D): 0.5

Figure 3.19: Closed-loop thermal control system modeling in Simulink/Simscape.

### 3.5.3 Hybrid Motion Control Logic

## In addition to thermal stability, the system demands stringent kinematic precision to ensure consistent productivity [21]. A two-phase motion strategy was innovated for the stepper motors:

## Blind Zone Phase: The packaging film is pulled at maximum speed utilizing the hardware RMT peripheral. This generates highly precise waveforms without consuming CPU cycles, thereby maximizing system efficiency [16].

## Search Zone Phase: The system transitions into a precise software-stepping mode to poll the registration "eye-mark" sensor after every individual microstep, ensuring the alignment of the bag with the cutting blade with micrometric accuracy [18].

### 3.5.4 Software Architecture of the Finite State Machine (FSM)

## The operational firmware was structured based on a Finite State Machine (FSM) model, providing deterministic stability when transitioning between packaging phases [21]:

## Idle State: Actively monitors the sealing jaw temperature and the salt level within the hopper.

## Parallel Pull & Fill State: Concurrently executes the film pulling and auger rotation to minimize the overall cycle time [21].

## Seal & Cut State: Actuates the pneumatic valves based on precisely defined timing intervals.

### 3.5.5 Industrial Isolation and Signal Integrity

## Given the severe electromagnetic noise generated by the heaters and motors, the hardware interface was designed to ensure the highest degree of reliability:

## Galvanic Isolation: Optocouplers are utilized to separate the sensitive 3.3V control circuits from the 24V motors and pneumatic valves.

## Zero-Crossing Thermal Control: The implementation of a Solid State Relay (SSR) featuring Zero-Crossing prevents electrical grid interference when applying the time-proportional control frequencies generated by the PID controller.

### 3.5.6 Industrial Reliability and Protection Protocols (Fail-Safe Logic)

## The control system is engineered to withstand continuous thermal and mechanical disturbances without inducing undesirable oscillations or ringing [14, 15]. To guarantee this, stringent software and hardware protection layers were integrated:

## Emergency Stop (E-Stop) System: The emergency button is hardwired to a hardware Interrupt Service Routine (ISR) that preempts all system tasks, immediately halting motor pulses and de-energizing the SSR relays [17].

## Integrator Anti-Windup Clamping: A saturation algorithm was embedded to prevent the excessive accumulation of integral error in the PID equation during the cold-boot heating phase. Simulation results of the closed-loop step response demonstrated the efficiency of this algorithm, achieving a settling time of 225.3 seconds with a negligible, near-zero overshoot of 0.3041%. The peak temperature reached only 150.45°C, confirming absolute system stability at the 150°C setpoint [14, 15].

## Sensor Failure Cutoff: The system immediately cuts power to the heaters if the SPI bus reports a failure in reading the thermocouple data.

## Over-Temperature Protection: A hard ceiling for temperature was programmed, which completely shuts down the system if exceeded, ensuring the Teflon tape does not burn and the sealing jaws are protected from thermal damage.

## 3.7 Summary of Design Specifications

Table 3.2 summarizes the key design specifications for the VFFS auger filling system:

Table 3. 2:Summary of Design Specifications

| Parameter | Specification | Unit |
| --- | --- | --- |
| Production Rate | 20 | PPM |
| Package Weight | 500 | g |
| Weight Accuracy | ±1.5 | % |
| Filling Time | 2.0 | s |
| Auger Diameter | 60 | mm |
| Auger Pitch | 60 | mm |
| Shaft Diameter | 16 | mm |
| Radial Clearance | 1.5 | mm |
| Operating Speed | 300 | RPM |
| Motor Power | 400 | W |
| Gear Ratio | 20:1 | - |
| Hopper Capacity | 20 | L |
| Agitator Speed | 30 | RPM |
| Material | AISI 304 | - |
| Surface Finish | Ra < 0.8 | μm |

The design specifications presented in this chapter provide the foundation for the analytical modeling and numerical simulation presented in subsequent chapters. The integration of mechanical design, control system architecture, and VFFS compatibility ensures that the system meets all target performance requirements.

## 3.6 Electronic Control System Design

### 3.6.1 System Architecture Overview

The electronic control system for the Vertical Form-Fill-Seal (VFFS) packaging machine is designed as a three-layer hierarchical architecture to ensure reliable operation in industrial environments. This architecture separates logic control, power interfacing, and high-power actuation to protect sensitive components from electrical noise and transients.

The first layer comprises the main controller (ESP32 microcontroller) operating at 3.3V DC, handling all computational tasks, sensor data acquisition, and communication protocols. The second layer consists of power interface modules including stepper motor drivers (A4988), Darlington transistor arrays (TIP122) for solenoid control, and solid-state relays for heating elements. These modules translate low-voltage logic signals into power signals capable of driving actuators. The third layer contains the high-power loads including stepper motors, pneumatic solenoid valves, and resistive heating elements operating at 12V-24V DC and 220V AC.

This layered approach provides galvanic isolation between the sensitive microcontroller and the noisy industrial power environment, ensuring signal integrity and system longevity.

### 3.6.2 Signal Isolation and Conditioning

#### 3.6.2.1 Optical Isolation Using PC817

Industrial packaging machinery generates significant electromagnetic interference (EMI) from motor switching, solenoid actuation, and heater cycling. To protect the ESP32 from these disturbances, optical isolation is implemented for all sensor inputs using PC817 optocouplers.

The PC817 provides 5kV isolation between input and output sides, with a Current Transfer Ratio (CTR) ranging from 50% to 600%. The input side operates at 5V with a series current-limiting resistor, while the output side interfaces with the 3.3V logic of the ESP32.

1Figure 3.1: PC817 Optocoupler

As you see in Figure 3.1 Optical isolation circuit using PC817 optocoupler

The input current is calculated to ensure reliable LED operation without excessive power dissipation:

IF = (Vsupply - VF_LED) / Rinput

IF = (5V - 1.2V) / 220 Ohm

IF = 3.8V / 220 Ohm

IF = 0.0173 A = 17.3 mA

Where:

IF = Forward current through LED (17.3 mA)

Vsupply = Supply voltage (5V)

VF_LED = LED forward voltage drop (1.2V)

Rinput = Current limiting resistor (220 Ohm)

This current provides sufficient illumination for reliable phototransistor activation while maintaining LED longevity. The 10k Ohm pull-up resistor on the output side ensures defined logic levels, providing approximately 0.33mA collector current when the transistor saturates, yielding a low logic level below 0.3V.

| Parameter | Symbol | Value | Unit | Notes |
| --- | --- | --- | --- | --- |
| Isolation Voltage | V\_ISO | 5000 | V | Between input and output |
| Forward Current | I\_F | 17.3 | mA | Calculated for LED operation |
| Current Transfer Ratio | CTR | 100 | % | Typical value at 25 deg C |
| Output Pull-up Resistor | R\_pullup | 10 | kOhm | Ensures defined logic high |
| Response Time | t\_r | 18 | Us | Suitable for industrial control |
| Collector Current | I\_C | 0.33 | mA | At saturation, V\_CE approx 0.2V |

1Table 3.1: PC817 Optocoupler Specifications and Operating Parameters

#### As you see in Table 3.1 Operating parameters for PC817 optocoupler in sensor interface applications, showing calculated values for current limiting and pull-up resistors to ensure reliable signal transmission with 3.3V logic compatibility.

#### 3.6.2.2 Sensor Interface Circuits

Two primary sensors require optical isolation: the Eye Mark sensor for film registration and the Hopper Level sensor for salt quantity monitoring.

The Eye Mark sensor interface utilizes GPIO39 on the ESP32, with the optocoupler output configured as active-low. When the sensor detects the registration mark on the packaging film, the phototransistor conducts, pulling GPIO39 to logic low. This configuration provides noise immunity for the critical film positioning function.

The Hopper Level sensor employs GPIO35 with identical isolation circuitry. This sensor prevents dry-running of the auger motor by detecting low salt levels, protecting mechanical components from damage.

### 3.6.3 Temperature Measurement System

#### 3.6.3.1 MAX6675 Thermocouple Interface

Precise temperature control is essential for heat-sealing operations in salt packaging. The system employs Type-K thermocouples interfaced with MAX6675 cold-junction-compensated thermocouple-to-digital converters. This integrated circuit eliminates the need for analog signal conditioning by providing direct digital temperature readings via Serial Peripheral Interface (SPI) protocol.

The MAX6675 operates with 12-bit resolution (0.25 deg C per count) over a range of 0 deg C to 1024 deg C, sufficient for sealing jaw applications typically operating at 150-250 deg C. Cold-junction compensation is performed internally, correcting for temperature variations at the connection terminals.

3Figure 3.2: MAX6675 SPI

As you see in Figure 3.3  Oscilloscope capture of SPI communication between ESP32 and MAX6675 temperature converter, showing clock signal (SCK, 100 kHz), chip select activation (CS), and serial data output (SO) with 16-bit temperature data frame.

The SPI interface connections are configured as follows:

| MAX6675 Pin | ESP32 GPIO | Function | Description |
| --- | --- | --- | --- |
| SCK | GPIO18 | Serial Clock | Synchronization signal for data transfer |
| SO | GPIO19 | Serial Output | Temperature data from MAX6675 to ESP32 |
| CS (Vertical) | GPIO22 | Chip Select 1 | Selects vertical sealing jaw thermocouple |
| CS (Horizontal) | GPIO21 | Chip Select 2 | Selects horizontal sealing jaw thermocouple |
| VCC | 3.30V | Power Supply | Logic supply voltage |
| GND | GND | Ground | Common reference |

2Table 3.2: MAX6675 SPI Interface Pin Assignment

As you see in Table 3.2  SPI bus pin assignment for dual MAX6675 temperature measurement system, utilizing shared clock and data lines with individual chip select signals for independent reading of vertical and horizontal sealing jaw temperatures.

The temperature calculation from raw SPI data follows the manufacturer's specification:

T_Celsius = (Raw_Data >> 3) / 4

Where:

T_Celsius = Temperature in degrees Celsius

Raw_Data = 16-bit value read from MAX6675

3 = Right shift by 3 bits (removes status flags)

/ 4 = Division by 4 (accounts for 0.25 deg C resolution)

3.6.4 Power Actuator Drivers

3.6.4.1 Solenoid Valve Drivers (TIP122)

Pneumatic actuation for the cutting knife and sealing jaws requires driving inductive loads (solenoid coils) from 3.3V logic signals. The TIP122 Darlington pair transistor is selected for this application due to its high current gain (hFE >= 1000) and ability to switch currents up to 5A.

The Darlington configuration provides two-stage current amplification, allowing the ESP32 GPIO (maximum 12mA source current) to control solenoids drawing 0.5-1A at 24V DC.

5Figure 3.3: TIP122

As you see in Figure 3.3  TIP122 Darlington transistor solenoid driver circuit

Base resistor calculation ensures sufficient drive current for saturation:

R_base = (V_GPIO - V_BE_total) / I_B

Where:

R_base = Base resistor value (Ohm)

V_GPIO = GPIO output voltage (3.3V)

V_BE_total = Total base-emitter voltage drop (1.4V)

I_B = Required base current (A)

The base current is calculated with safety factor:

I_B = I_C / (hFE x SF)

I_B = 1A / (1000 x 1.5)

I_B = 1A / 1500

I_B = 0.00067 A = 0.67 mA

Where:

I_B = Base current (A)

I_C = Collector current (1A, solenoid nominal current)

hFE = Current gain (1000)

SF = Safety factor (1.5)

Yielding:

R_base = (3.3V - 1.4V) / 0.67 mA

R_base = 1.9V / 0.00067 A

R_base = 2836 Ohm = 2.8 kOhm

A standard value of 1 kOhm is selected to provide adequate margin while protecting the ESP32 GPIO from overcurrent conditions.

| Parameter | Symbol | Value | Unit | Calculation/Notes |
| --- | --- | --- | --- | --- |
| Collector Current | I\_C | 1.0 | A | Solenoid nominal current |
| Current Gain | hFE | 1000 | - | Minimum Darlington gain |
| Safety Factor | SF | 1.5 | - | Ensures deep saturation |
| Base Current | I\_B | 0.67 | mA | I\_C / (hFE x SF) |
| Base Resistor | R\_base | 1.0 | kOhm | Standard value with margin |
| Saturation Voltage | V\_CE\_sat | 0.2 | V | Typical at 1A collector current |
| Power Dissipation | P\_D | 0.2 | W | I\_C x V\_CE\_sat |

3Table 3.3: TIP122 Solenoid Driver Operating Parameters

Table 3.3 Caption: Operating parameters for TIP122 Darlington driver in solenoid valve control application, showing calculated base current with safety factor and resulting power dissipation requiring minimal heatsinking.

Critical to reliable operation is the flyback diode (1N4007) placed in anti-parallel with each solenoid coil. When the transistor switches off, the collapsing magnetic field generates a voltage spike (back-EMF) potentially exceeding 100V. The diode provides a recirculation path for this energy, clamping the voltage at approximately 0.7V and protecting the transistor from avalanche breakdown.

3.6.4.2 Stepper Motor Drivers (A4988)

Precise film feeding and auger metering require stepper motor control with microstepping capability. The A4988 stepper motor driver provides up to 1/16 microstepping, translating 200 full steps per revolution into 3200 microsteps for smooth motion and accurate positioning.

The A4988 incorporates internal current regulation, allowing motor current to be set via a sense resistor and reference voltage. This feature enables torque optimization while preventing motor overheating during continuous operation.

7Figure 3.4: A4988 Stepper Driver Interface

As you see in Figure 3.4 A4988 stepper motor driver interface showing STEP/DIR/ENABLE connections .

Current limiting is essential for proper operation. The reference voltage and sense resistor determine the chopping current:

I_chop = V_ref / (8 x R_sense)

For the selected sense resistor of 43 mOhm and desired current of 1.5A:

V_ref = I_chop x 8 x R_sense

V_ref = 1.5A x 8 x 0.043 Ohm

V_ref = 1.5 x 8 x 0.043

V_ref = 0.516 V

This reference voltage is set via the onboard potentiometer on the A4988 module.

| Parameter | Film Feed Motor | Auger Motor | Unit | Notes |
| --- | --- | --- | --- | --- |
| STEP Pin | GPIO32 | GPIO25 | - | Step pulse input |
| DIR Pin | GPIO33 | GPIO26 | - | Direction control |
| ENABLE Pin | GPIO14 | GPIO27 | - | Active-low enable |
| Current Limit | 1.5 | 1.5 | A | Per phase |
| Sense Resistor | 43 | 43 | mOhm | Standard value |
| Reference Voltage | 0.516 | 0.516 | V | Set by trimpot |
| Microstepping | 1.0/16 | 1.0/16 | - | 3200 steps/revolution |
| Pull-down Resistor | 100 | 100 | kOhm | Ensures disabled at startup |

4Table 3.4: A4988 Stepper Driver Configuration

Table 3.4 Caption: A4988 stepper driver configuration parameters for film feeding and auger metering systems, showing identical current settings with dedicated GPIO assignments and pull-down resistors for safe startup state.

The ENABLE pins incorporate 100 kOhm pull-down resistors to ensure motors remain de-energized during ESP32 boot-up, preventing uncontrolled motion before firmware initialization completes.

3.6.5 Solid State Relay Interface

Heater control for sealing jaws utilizes a Solid State Relay (SSR) interfaced to GPIO23. The SSR provides zero-cross switching to minimize electromagnetic interference and offers optical isolation between the 3.3V control circuit and the 220V AC heater load.

The interface includes a 330 Ohm current-limiting resistor for the SSR's internal LED and a 100 kOhm pull-down resistor to ensure heater remains off during system startup.

8Figure 3.5: SSR Control

As you see in Figure 3.5 Solid State Relay control interface

### 3.6.6 PCB Design and Layout Considerations

#### 3.6.6.1 Trace Width Calculations

Power distribution traces are sized according to IPC-2221B standards to prevent excessive heating and voltage drop. For the 12V motor supply carrying 2A maximum current, with 1oz copper thickness and 10 deg C temperature rise:

W = I / (k x delta_T^0.44)

W = 2 / (0.048 x 10^0.44)

W = 2 / (0.048 x 2.75)

W = 2 / 0.132

W = 0.76 mm

Where:

W = Minimum trace width (mm)

I = Current (2A)

k = Constant (0.048 for external layers)

delta_T = Temperature rise (10 deg C)

^0.44 = Raised to power of 0.44

A conservative width of 1.2mm is selected to provide safety margin and accommodate current spikes during motor acceleration.

| Net | Current (A) | Calculated Width (mm) | Selected Width (mm) | Notes |
| --- | --- | --- | --- | --- |
| VMOT (12V) | 2.0 | 0.76 | 1.20 | Motor supply with safety margin |
| VCC (5V) | 0.5 | 0.30 | 0.60 | Sensor supply |
| VDD (3.3V) | 0.3 | 0.20 | 0.40 | Logic supply |
| Signal | 0.01 | 0.15 | 0.30 | General purpose |
| GND Plane | - | - | Full pour | Return paths for all signals |

5Table 3.5: PCB Trace Width Specifications

Table 3.5 Caption: PCB trace width calculations per IPC-2221B standard for 1oz copper with 10 deg C temperature rise, showing conservative design margins for reliability in industrial environments.

#### 3.6.6.2 Thermal Management

Heat-generating components (A4988 drivers, TIP122 transistors) require thermal management to prevent junction temperatures exceeding maximum ratings. The A4988 in particular dissipates significant power during continuous operation:

P_diss = 2 x I_phase^2 x R_DS_on

P_diss = 2 x (1.5A)^2 x 0.32 Ohm

P_diss = 2 x 2.25 x 0.32

P_diss = 1.44 W

Where:

P_diss = Power dissipation (W)

I_phase = Phase current (1.5A)

R_DS_on = MOSFET on-resistance (0.32 Ohm)

^2 = Squared

Thermal vias (0.3mm diameter, plated) connect the exposed pad to copper pours on both PCB layers, providing heat spreading. With thermal resistance of approximately 30 deg C/W for the PCB arrangement, junction temperature is estimated as:

T_j = T_a + (P_diss x R_th_JA)

T_j = 50 deg C + (1.44W x 30 deg C/W)

T_j = 50 + 43.2

T_j = 93.2 deg C

Where:

T_j = Junction temperature (deg C)

T_a = Ambient temperature (50 deg C)

R_th_JA = Junction-to-ambient thermal resistance (30 deg C/W)

x = Multiplication

This remains below the 150 deg C maximum junction temperature with adequate margin.

#### 3.6.6.3 Electromagnetic Compatibility

Noise suppression measures include:

100nF ceramic decoupling capacitors at each IC power pin

100uF electrolytic bulk capacitors at motor supply entry points

Ferrite beads on thermocouple leads for high-frequency filtering

Ground plane continuity with minimal slotting

Separation of high-current motor traces from sensitive analog signals

9Figure 3.6: PCB Layer Stack-up and Grounding Strategy

As you see in Figure 3.6  PCB layer stack-up showing top signal layer, internal ground plane, internal power plane, and bottom signal layer configuration with via stitching for ground continuity and electromagnetic interference reduction.

#### 3.6.7 Power Budget and Distribution

| Rail | Voltage (V) | Current (A) | Power (W) | Duty Cycle | Avg. Power (W) |
| --- | --- | --- | --- | --- | --- |
| VMOT (Motors) | 12 | 5.0 | 60.0 | 50.0% | 30.0 |
| VCC (Sensors) | 5 | 0.04 | 0.2 | 100.0% | 0.2 |
| VDD (Logic) | 3.3 | 0.14 | 0.45 | 100.0% | 0.45 |
| Heater (SSR) | 220 | 2.0 | 440.0 | 25.0% | 110.0 |
| **Total** | - | - | **500.65** | - | **140.65** |

6Table 3.6: System Power Budget Analysis

As you see in Table 3.6  Comprehensive power budget for VFFS packaging machine showing peak and average power consumption across all voltage rails, with heater duty cycle based on thermal regulation requirements and motor duty cycle based on typical packaging cycle timing.

The selected power supply (MeanWell LRS-120-12, 120W) provides 15.96x margin over average consumption and 2.4x margin over peak non-heater loads, ensuring reliable operation with adequate headroom for inrush currents and supply degradation over time.

### 3.6.8 Pneumatic System Design:

VFFS pneumatic settings for different materials:

| Component | Fiber 85 |  | Fiber 120 | Plastic 15-35 | Plastic 50 |
| --- | --- | --- | --- | --- | --- |
| Seal jaws |  | 6 bars | 6 bars | 6 bars | 6 bars |
| Length seal |  | 5 bars | 5 bars | 5 bars | 5 bars |
| Belt transport |  | 5 bars | 5 bars | 5 bars | 6 bars |
| Film brake |  | 3 bars | 3 bars | 3 bars | 3 bars |
| Film tension |  | 2 bars | 1 bars | 0 bars | 4 bars |

Table:3.3.1 1:VFFS pneumatic settings for different materials

### 3.6.9 Safety and Protection Systems

#### 3.6.8.1 Emergency Stop Implementation

Hardwired emergency stop functionality is implemented via GPIO34, configured as external hardware interrupt. The normally-closed mushroom button provides fail-safe operation: any wire break or button press immediately triggers interrupt service routine bypassing RTOS scheduler, forcing immediate stepper stop (RMT channel termination), heater disable (SSR off), and pneumatic valve closure.

Response time is specified below 50ms from button press to actuator de-energization, meeting ISO 13850 Category 0 stop requirements.

#### 3.6.8.2 Component Protection

Transient voltage suppression is implemented at multiple levels:

Metal Oxide Varistors (MOVs) on 24V power entry for surge protection

TVS diode arrays (SMBJ series) on 3.3V logic lines

Flyback diodes (1N4007) on all inductive loads

Fuse protection (F1-F3) for individual actuators

| Location | Component | Rating | Function |
| --- | --- | --- | --- |
| 24V Input | MOV | 30.0V | Surge voltage clamping |
| 3.3V Logic | TVS Array | 5.0V | ESD and transient protection |
| Solenoid Coils | 1N4007 | 1A, 1000V | Back-EMF recirculation |
| Motor Drivers | Internal | 1.50A | Overcurrent limiting (A4988) |
| Heater Circuit | Fuse | 10.0A | Overcurrent protection |
| System Input | Circuit Breaker | 16.0A | Manual disconnect, short circuit |

7Table 3.7: Protection Component Summary

As you see in Table 3.7 : Protection component summary showing multi-level approach to system safety including surge suppression, transient voltage protection, inductive kickback prevention, and overcurrent protection for all critical circuits.