# Academic Graduation Project Proposal: Design and Development of a Vertical Form Fill Seal (VFFS) Powder Packaging Machine with Advanced Mechatronic Control

## Section A

### 1.1 Executive Summary

This proposal outlines a detailed plan for the design and construction of a working prototype for an Automated Vertical Form Fill Seal (VFFS) machine, specifically engineered for powder packaging utilizing an Auger Filler system.1 The system aims to fully automate the entire packaging cycle—starting from film unwinding and bag forming, through precise powder dosing, to heat sealing and cutting—all under the command of an advanced control architecture (PLC/Servo).3 The primary engineering challenge of this project is the design and implementation of a Gravimetric Feedback Loop to dynamically compensate for variations in powder density, thereby guaranteeing high accuracy in the final package weight.4 The project seeks to significantly enhance production efficiency, reduce material wastage from overfilling, and ensure consistent product quality compared to conventional volumetric filling methods.6 Deliverables will include a functional prototype, comprehensive engineering documentation, and quantitative performance data demonstrating its feasibility for educational and small-scale industrial applications.

### 1.1.1 Key Objectives

The core objectives of this project are:

- To automate the complete VFFS cycle, encompassing film pulling, vertical and horizontal sealing, and the Auger Dosing system.7

- To develop an advanced mechatronic control system utilizing a PLC and Servo Drives to ensure highly accurate synchronization for the intermittent motion required during the filling process.8

- To design and implement a closed-loop control system (Gravimetric Feedback) that uses weight data from a Checkweigher (or equivalent) to continuously adjust the Auger rotation count, thus maximizing filling accuracy.4

- To engineer and integrate active Flow Aids, such as vibrators, to maintain smooth powder flow and prevent common mechanical issues like bridging inside the filling hopper.11

- To incorporate essential safety interlocks (Safety Interlocks) and ensure compliance with hygienic design standards (e.g., using stainless steel for product contact parts).13

- To validate performance through controlled testing, measuring key metrics such as weight accuracy (within a defined $\pm$ percentage) and production rate (bags/minute).14

### 1.1.2 Features of the Production Line

The proposed production line will incorporate the following key mechatronic components:

- Film Pulling and Transport Unit: Driven by a Servo Motor for precise film tension control and accurate bag length determination, integrated with a Registration Sensor for print mark tracking and alignment.15

- Auger Filler Mechanism: A volumetric dosing system controlled by a dedicated motor (preferably a servo for precision) designed to dispense a predetermined amount of powder.16

- Flow Aid System: Pneumatic or electric vibrators and agitators, automatically activated by PLC Timed Outputs 12 and level sensors, to manage powder flow within the hopper.18

- Cross Sealing Jaws: Actuated by Servo Motor Drive 3 or Pneumatic Cylinder 10 for high-speed, precise closure, equipped with heating elements and a PID control system to ensure consistent thermal sealing.19

- Control Panel: A centralized PLC control system 21 with a Human-Machine Interface (HMI) touch screen for malfunction display, parameter input (timing, temperature, auger counts), and sequence logic control.

### 1.1.3 Financial Overview

The following table provides initial cost estimates for the primary components necessary to build an advanced prototype suitable for a Mechatronics project:

| Item | Description (Example) | Estimated Cost (USD) |
| --- | --- | --- |
| 1. Main Control Unit | Advanced PLC/Microcontroller + HMI | 100-250 |
| 2. Servo Motion System | Servo Motor (for film pull) and Drive | 200-400 |
| 3. Heating & Control Elements | Heating elements + Thermal Sensors (RTD) + PID controllers | 80-150 |
| 4. Mechanical Components | Stainless Steel (Hopper, Auger) + Structural Frame Materials | 150-300 |
| 5. Flow Aid Systems | Electric/Pneumatic Vibrators and Valves | 50-100 |
| Total (Approximate) | Includes higher end for Mechatronic Application | 580-1200 |

### 1.1.4 Summary

The Automated Auger VFFS machine project will demonstrate that the synergistic integration of advanced mechatronics—specifically Servo Control, Gravimetric Feedback, and precise PID thermal control—can effectively solve key challenges in traditional packaging processes, such as weight inconsistency and low sealing quality.22 Building and testing this prototype will validate the proposed design choices and quantify the improvement in filling accuracy and overall cycle speed compared to a standard volumetric filling system.

## Section B

### 2.1 History

The history of Vertical Form Fill Seal (VFFS) technology dates back to Walter Zwoyer's patent in 1936.23 VFFS machines are the preferred choice for packaging non-uniform, bulk, granular, and powder products because they leverage gravity for efficient filling.24 The Auger Filler mechanism was specifically developed for the precise dosing of non-free-flowing powders, such as flour or chemical detergents, while effectively controlling dust emission during the measuring process.2 Modern advancements have shifted VFFS control from simple pneumatic/mechanical systems to complex digital control incorporating Servo Motors to achieve the high-speed and precision required for intermittent motion applications common in powder packaging.26

### 2.2 Problem Definition

In many industrial and small-scale operations, volumetric powder filling using Auger systems suffers from several critical challenges:

- Inconsistent Weight: Volumetric measurement is inherently inaccurate when powder density changes (a common occurrence), leading to deviations in the final package weight. This results in material waste (overfilling) or non-compliance (underfilling).2

- Flow Interruption: Powders frequently encounter mechanical issues in the hopper, such as bridging (material interlocking to form an arch) or rat-holing (material only flowing down the center channel), which causes production downtime and necessitates manual intervention.27

- Sealing Errors: Weak seals or film burn-through are common due to inconsistent pressure, inadequate heat control, or contamination of the sealing jaws, jeopardizing product quality and shelf life.28

- Speed and Synchronization Limits: VFFS machines for powders require intermittent motion (full stop for filling), which demands high-speed synchronization between film movement and jaw closure to achieve optimal production rates.8

The proposed solution is to develop a smart mechatronic system that integrates real-time weight sensing (Checkweigher) with motion control (Servo Control) and active flow management (Flow Aids) to minimize these errors, enhance safety, and significantly increase repeatability and accuracy.

### 2.3 Overall and Specific Objectives

Overall Objective:

To develop an automated prototype VFFS machine with an Auger Filler that demonstrates a substantial improvement in weight accuracy and production efficiency over conventional volumetric systems, by implementing advanced mechatronic control principles.14

Specific Objectives:

- Implement a Servo-based Motion Control system to precisely coordinate the timing of film pull and cross-jaw actuation.3

- Design and program the Gravimetric Feedback Algorithm within the PLC to receive real-time weight data and automatically adjust the auger's pulse count accordingly.4

- Design and integrate an effective hopper mechanism that includes PLC-controlled vibrators (Flow Aids) and agitators to consistently break powder agglomerations.11

- Apply a digital PID control system to maintain cross-sealing jaw temperature consistency within a narrow band of $\pm 2^{\circ} \text{C}$.19

- Evaluate the final mechatronic performance based on measured dosing accuracy, production speed (bags/minute), and seal integrity.

### 2.4 Targeted Activities and Outputs

#### 2.4.1 Targeted Activities

- Literature Review and Theoretical Analysis: Comprehensive study of VFFS techniques, Auger Filler principles, and closed-loop control theory for gravimetric feedback systems.2

- Mechanical Design (CAD): 3D modeling of the Auger mechanism, hopper assembly, film transport system, and sealing jaw holder, including precise calculations for material selection (Stainless Steel) and component dimensions.31

- Control System Development: Creation of the PLC logic flowchart (Sequence Logic), selection of electrical components (servos, drives, sensors, relays), and development of the control programs (including synchronization and weight correction algorithms).32

- Fabrication and Assembly: Manufacturing of the structural frame and mechanical parts, assembly of subsystems, and electrical wiring of the control panel.33

- Configuration and Testing: Servo motor tuning 34, PID loop tuning for thermal control 35, initial functional testing, debugging, and resolving motion synchronization errors.28

#### 2.4.2 Targeted Outputs

- CAD Drawings & Bill of Materials (BOM): Detailed mechanical designs and a comprehensive list of all required electrical and mechanical components.

- Mechatronic Control Software: Fully documented PLC code with HMI programming, integrating motion control, temperature control, and the weight correction algorithm.

- VFFS Prototype Machine: A fully functional, automated VFFS system capable of packaging powders.

- Performance and Analysis Report: An experimental report containing quantitative data documenting weight accuracy (comparing volumetric vs. gravimetric feedback) 6, cycle speed, and seal consistency.

### 2.5 Beneficiaries

#### 2.5.1 Direct Beneficiaries

- Team Members and Mechatronics Students: Acquisition of advanced practical skills in system integration (mechanics/electronics/control) and hands-on experience with modern industrial technologies like Servo Motion and closed-loop control systems.

- Educational Laboratories and Universities: Provision of a low-cost, yet sophisticated, educational platform to demonstrate industrial automation principles, advanced dosing systems, and feedback control.

- Small Workshops and Startups: Benefit from an accurate, reliable, and cost-effective powder packaging solution, enabling them to comply with legal weight requirements and minimize material waste.22

#### 2.5.2 Indirect Beneficiaries

- Faculty and Researchers: The prototype can serve as a research tool for further applied studies in advanced control algorithm optimization (e.g., implementing simple machine learning for dosing refinement).4

- Local Industries (Food and Chemical): Supporting localized manufacturing by providing a high-efficiency automation technology capable of accurately handling diverse powders.

- The General Public: Assurance of product quality and integrity through improved packaging accuracy and adherence to hygiene and quality standards (CGMP/FDA).13

### 2.6 Impact and Impact Pathways

#### 2.6.1 Impact

- Improved Quality and Accuracy: Achieving higher levels of weight accuracy in powder filling through dynamic compensation of material density changes.

- Increased Productivity and Efficiency: Higher packaging throughput due to the use of servo drives for fast intermittent motion and the elimination of production stops caused by bridging.

- Enhanced Safety and Compliance: Designing the machine in accordance with industrial safety standards (e.g., AS 4024) 13 and hygiene requirements (Stainless Steel parts).31

#### 2.6.2 Impact Pathways

- Gravimetric Feedback System: The closed-loop control will automatically regulate dosing based on measured weight, directly mitigating material density variations and reducing human/material weighing errors.6

- Synchronized Mechatronic Control: Servo control ensures high-speed synchronization between film pull and cross-jaw sealing, which reduces the overall packaging cycle time.

- Automated Flow Control: The PLC will automatically activate vibrators on the hopper upon sensing flow interruption or slowdown, ensuring a continuous and reliable powder supply.12

### 2.7 Relevance

This project addresses a clear industrial need for cost-effective and accurate powder packaging machines, a necessity in the food and pharmaceutical sectors.37 It also directly aligns with global trends toward integrating "Industry 4.0" concepts and advanced mechatronics applications in automation, through the use of Programmable Logic Controllers (PLC), Servo systems, and establishing the groundwork for potential advanced algorithm implementation in dosing control.3

## Section C

### 3.1 Methodology

The project will be executed following a standard mechatronic system design methodology, encompassing five main phases:

- Design and Analysis Phase: Defining engineering requirements (target packaging speed, required weight accuracy, film type). Creating CAD models and the control flowchart. Conducting precise Motor Sizing calculations, particularly focusing on Inertia Analysis for the cross-jaw mechanism.34

- Procurement & Fabrication Phase: Purchasing all electrical components (PLC, HMI, Servo drives, sensors). Fabricating the structural frame and mechanical parts, ensuring proper finishing for stainless steel parts.

- Control Implementation Phase: Wiring the electrical system and verifying circuit integrity. Programming the core PLC sequence logic and HMI integration. Configuring and tuning the servo motors and implementing PID control for the thermal system.35

- Integration and Initial Testing Phase: Assembling all subsystems and conducting individual function validation tests (sealing test, film pull test, auger rotation test). Running the full packaging cycle validation and troubleshooting synchronization errors.28

- Evaluation and Performance Phase: Conducting quantitative experiments to measure weight accuracy under various powder conditions. Comparing filling accuracy between standard volumetric mode and the Gravimetric Feedback mode. Documenting final production rates and overall performance in the Final Test Report.

Preliminary Timeline (6-8 months)

Month 1: In-depth Research and Engineering Analysis

Month 2: Detailed CAD Design, Inertia Calculations, and Procurement

Month 3: Mechanical Assembly (Frame, Auger, Hopper)

Month 4: Control System Setup, PLC/HMI Programming, and Servo Configuration

Month 5: Integration and Initial Functional Testing (Synchronization, Thermal PID)

Month 6: Accuracy Validation (Gravimetric Feedback) and Final Documentation

## Section D

### 4.1 Conclusion

The proposed project will deliver a functional prototype of an automated VFFS machine with an advanced Auger Filler system. It will successfully demonstrate significant improvements in measurement accuracy and operational efficiency through the implementation of core mechatronic principles, such as gravimetric control and advanced motion synchronization. The final documentation and test results will affirm the necessity of integrating these technologies to enhance the powder packaging process, providing valuable insights for both academia and specialized industrial applications.6

### 4.2 References

(Full academic references will be compiled according to the approved format in the final project report).

#### المصادر التي تم الاقتباس منها

- Vertical Form Fill Seal Machine:The Complete Buying Guide in 2025 -, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.allpackchina.com/vertical-form-fill-seal-machine/

- أوجير حشو-آلة-أخبار-شانغهاي داخه التعبئة الآلية المحدودة, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.dahepacking.com/news/auger-filler-17976898.html

- VFFS GP480PE - Great Pack, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.greatvffs.com/product-item/vffs-gp480pe/

- Machine Learning implementation on a machine : r/PLC - Reddit, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.reddit.com/r/PLC/comments/1idk9sh/machine_learning_implementation_on_a_machine/

- كيفية حل المشاكل الشائعة في آلة التعبئة والتغليف العمودية؟ - Smartpack, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://smartpackmc.com/ar/%D8%A7%D8%B3%D8%AA%D9%83%D8%B4%D8%A7%D9%81-%D8%A3%D8%AE%D8%B7%D8%A7%D8%A1-%D8%A2%D9%84%D8%A9-%D8%A7%D9%84%D8%AA%D8%B9%D8%A8%D8%A6%D8%A9-%D8%A7%D9%84%D8%B9%D9%85%D9%88%D8%AF%D9%8A%D8%A9-%D9%88%D8%A5%D8%B5%D9%84%D8%A7%D8%AD%D9%87%D8%A7/

- Optimizing Filling Processes with Checkweighing Equipment - Mettler Toledo, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.mt.com/us/en/home/library/know-how/product-inspection/feedback-control.html

- Application Technique - Vertical Form Fill and Seal - Rockwell Automation, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://literature.rockwellautomation.com/idc/groups/literature/documents/at/motion-at002_-en-p.pdf

- Continuous vs Intermittent VFFS Machines Explained, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://unifiedflex.com/faq/what-is-the-difference-between-continuous-and-intermittent-vffs-operation/

- TECHNOLOGY AND INNOVATION IN FOOD PACKAGING, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://thesis.unipd.it/bitstream/20.500.12608/24726/1/Conselvan_Alberto_1106211.pdf

- (PDF) Mechatronics Design of a Low-Cost Packaging and Dosing Machine for Doughy Products - ResearchGate, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.researchgate.net/publication/229040784_Mechatronics_Design_of_a_Low-Cost_Packaging_and_Dosing_Machine_for_Doughy_Products

- How to Keep Dry Ingredients Flowing With Production | POWER Engineers, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.powereng.com/library/how-to-keep-dry-ingredients-flowing-with-production/

- FLOW AIDS - OLI, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://app.olivibra.com/open/public/documents/475-Cat_FlowAids_CFAEN_0925.pdf

- Vertical(VFFS) - SaintyCo, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.saintyco.com/category/packing-solution/verticalvffs/

- Vertical Form Fill Seal Machine ROI: Real-World Performance Metrics - Wolf Packing, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://wolf-packing.com/vertical-form-fill-seal-machine-roi-real-world-performance-metrics/

- Vertical Form Fill Seal Machine (VFFS) - Guangdong Kenwei Intellectualized Machinery Co., Ltd., تم الوصول بتاريخ ‎نوفمبر 3, 2025، http://jwweigher.com/3-vertical-form-fill-seal-machine-vffs.html

- What is the working principle of auger filling machine? - Landpack, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.landpack.com/knowledge-center/what-is-the-working-principle-of--auger-filling-machine.html

- Powder caking or clumping review : causes and how to solve it - PowderProcess.net, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://powderprocess.net/Powder_Flow/Powder_Caking.html

- The Role of Material Flow Aids in Preventing Process Downtime - Erie Technical Systems, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.erietechnicalsystems.com/blog/the-role-of-material-flow-aids-in-preventing-process-downtime

- Seal Jaw for VFFS and HFFS Packaging Lines - Frigate Manufacturing, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://frigate.ai/en-za/product/packaging-machine-sealing-jaw/

- The Role of Temperature Control in VFFS Packaging, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://unifiedflex.com/faq/how-does-temperature-control-affect-vffs-packaging-consistency/

- PLC (Programmable Logic Controller) of Auger Filling Machine - TOPSPACK, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://topspack.com/plc-programmable-logic-controller-of-auger-filling-machine

- Design and Development of Cost Effective Automatic Machine For ..., تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.scribd.com/document/485121877/Paper87420-424

- Vertical form fill sealing machine - Wikipedia, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://en.wikipedia.org/wiki/Vertical_form_fill_sealing_machine

- Vertical vs. Horizontal Form Fill Systems: Key Differences - TDI Packsys, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.tdipacksys.com/blog/vertical-vs-horizontal-form-fill-system/

- VFFS مقابل. HFFS: أي آلة التغليف مناسبة لك? - Chlbpack - CHLB Packing Machine, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.chlbpack.com/ar/blog/vffs-vs-hffs-which-packaging-machine-is-right-for-you/

- Continuous vs Intermittent Motion in VFFS Machines - Unified Flex Packaging Technologies, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://unifiedflex.com/faq/how-do-continuous-and-intermittent-vffs-machines-differ/

- Solve Powder Bridging and Arching Problems in Silo and Hoppers - Matcon, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.matconibc.com/powder-handling-problems-solved-bridging

- Troubleshooting of VFFS Machine Problems - XFilPak, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://xfilpak.ca/troubleshooting-of-vffs-machine-problems/

- Troubleshooting the 11 Most Common Film Related Problems with a VFFS - Rovema NA, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.rovema-na.com/blog/troubleshooting-the-11-most-common-film-related-problems-with-a-vffs

- Evaluation and optimization of a vertical form, fill and seal production machine for flexible packaging papers - LUTPub, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://lutpub.lut.fi/bitstream/10024/161079/1/Evaluation%20and%20optimization%20of%20a%20vertical%20form%2C%20fill%20and%20seal%20production%20machine%20for%20flexible%20packaging%20papers.pdf

- High-Precision Auger Type Powder Filling Machines for Efficient Packaging, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.baopackmachinery.com/article/industry/detail/high-precision-auger-type-powder-filling-machines-for-efficient-packaging.html

- Vertical packaging machine, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.mitsubishielectric.co.jp/dl/fa/software/library/ssc/0000000050/o_file/bcn-b62005-665_a.pdf

- كيفية صنع ماكينة التعبئة – حلول آلات التعبئة المتخصصة, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://pacmastery.com/ar/how-to-make-a-packing-machine/

- Three Essential Factors for Servo Motor Sizing - Electromate Inc., تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.electromate.com/news/post/three-essential-factors-for-servo-motor-sizing

- PID Control Tutorial Series | 1. Control Temperature & Balance Both Target and Disturbance Response, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://www.youtube.com/watch?v=HXFU9sq3PqY

- PID Tuning Guide, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://tlk-energy.de/blog-en/practical-pid-tuning-guide

- VFFS Vertical Form Fill and Seal machines (Powder filling systems) - PowderProcess.net, تم الوصول بتاريخ ‎نوفمبر 3, 2025، https://powderprocess.net/Filling/VFFS.html