|  | Simulation of hopper2 Date: Tuesday, January 13, 2026 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Interaction Information 5 Mesh information 5 Sensor Details 5 Resultant Forces 6 Beams 6 Study Results 7 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: hopper2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Shell2; Solid Body; Mass:5.17315 kg Volume:0.00064203 m^3 Density:8,057.49 kg/m^3 Weight:50.6969 N; C:\Users\user1\Desktop\VFFS\التعبئة\التعبئة\assemply\hopper2.SLDPRT.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\user1\Desktop\VFFS\التعبئة\التعبئة\assemply).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; AISI 304. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.06807e+08 N/m^2. Tensile strength:; 5.17017e+08 N/m^2. Elastic modulus:; 1.9e+11 N/m^2. Poisson's ratio:; 0.29. Mass density:; 8,000 kg/m^3. Shear modulus:; 7.5e+10 N/m^2. Thermal expansion coefficient:; 1.8e-05 /Kelvin.; SolidBody 1(Shell2)(hopper2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 3.8594e-05; 657.529; 1.12057e-05; 657.529. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 2 face(s). Type:; Normal to selected face. Value:; 4,300. Units:; N/m^2. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Interaction Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 11.6625 mm. Minimum element size; 8.18419 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 39638. Total Elements; 19779. Maximum Aspect Ratio; 29.951. % of elements with Aspect Ratio < 3; 1.44. Percentage of elements with Aspect Ratio > 10; 5.18. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:04. Computer name:.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 3.8594e-05; 657.529; 1.12057e-05; 657.529. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 1.83284e-05; -0.00140002; -2.87592e-06; 0.00140014. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 9; 9.088e-04mm Node: 25479. hopper2-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Stress1; VON: von Mises Stress; 9.452e+03N/m^2 Node: 9091; 2.296e+06N/m^2 Node: 10760. hopper2-Static 1-Stress-Stress1. Name; Type; Min; Max. Factor of Safety1; Max von Mises Stress; 90.056 Node: 10760; 21,878.701 Node: 9091. hopper2-Static 1-Factor of Safety-Factor of Safety1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 4.278e-08 Node: 9091; 1.039e-05 Node: 10760. hopper2-Static 1-Strain-Strain1.

# Conclusion