|  | Simulation of auger and shaft Date: Tuesday, January 6, 2026 Designer: Solidworks Study name: auger and shaft analysis Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Interaction Information 5 Mesh information 5 Sensor Details 5 Resultant Forces 6 Beams 6 Study Results 7 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: auger and shaft Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Combine1; Solid Body; Mass:1.66362 kg Volume:0.000207953 m^3 Density:8,000.01 kg/m^3 Weight:16.3035 N; C:\Users\user1\Desktop\VFFS\auger and shaft analysis\auger and shaft.SLDPRT Jan 6 02:47:55 2026.

# Study Properties Study name; auger and shaft analysis. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\user1\Desktop\VFFS\auger and shaft analysis).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; AISI 304. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.06807e+08 N/m^2. Tensile strength:; 5.17017e+08 N/m^2. Elastic modulus:; 1.9e+11 N/m^2. Poisson's ratio:; 0.29. Mass density:; 8,000 kg/m^3. Shear modulus:; 7.5e+10 N/m^2. Thermal expansion coefficient:; 1.8e-05 /Kelvin.; SolidBody 1(Combine1)(auger and shaft). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 2 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -0.983818; -0.00516891; 0.000295244; 0.983831. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Torque-1; Reference:; Face< 1 >. Type:; Apply torque. Value:; -20 N.m..

# Connector Definitions No Data

# Interaction Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 6.81543 mm. Minimum element size; 0.340772 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 533776. Total Elements; 350589. Maximum Aspect Ratio; 62.851. % of elements with Aspect Ratio < 3; 99.2. Percentage of elements with Aspect Ratio > 10; 0.0108. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:38. Computer name:.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.983818; -0.00516891; 0.000295244; 0.983831. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.0305784; 0.016423; 0.0667354; 0.0752221. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 6.038e-03N/m^2 Node: 223160; 1.339e+08N/m^2 Node: 151154. auger and shaft-auger and shaft analysis-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 93; 3.203e-03mm Node: 7988. auger and shaft-auger and shaft analysis-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 3.904e-14 Element: 129294; 5.105e-04 Element: 169770. auger and shaft-auger and shaft analysis-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Max von Mises Stress; 1.544e+00 Node: 151154; 3.425e+10 Node: 223160. auger and shaft-auger and shaft analysis-Factor of Safety-Factor of Safety1.

# Conclusion