|  | Simulation of blenderaugerlong Date: Sunday, January 4, 2026 Designer: Solidworks Study name: Blade analysis long Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Interaction Information 5 Mesh information 6 Sensor Details 7 Resultant Forces 7 Beams 7 Study Results 8 Conclusion 10. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: blenderaugerlong Current Configuration: 默认. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line1; Solid Body; Mass:0.137864 kg Volume:1.7233e-05 m^3 Density:8,000 kg/m^3 Weight:1.35107 N; C:\Users\User01\Desktop\vffs\design\design\blenderaugerlong.SLDPRT Jan 4 01:27:30 2026.

# Study Properties Study name; Blade analysis long. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\User01\Desktop\vffs\design\design).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; AISI 304. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.06807e+08 N/m^2. Tensile strength:; 5.17017e+08 N/m^2. Elastic modulus:; 1.9e+11 N/m^2. Poisson's ratio:; 0.29. Mass density:; 8,000 kg/m^3. Shear modulus:; 7.5e+10 N/m^2. Thermal expansion coefficient:; 1.8e-05 /Kelvin.; SolidBody 2(Split Line1)(blenderaugerlong). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 3 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -1.97617; -2.95572; 0.000421062; 3.55549. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 5 face(s). Type:; Normal to selected face. Value:; 4,000. Units:; N/m^2. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Interaction Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 1.74415 mm. Minimum element size; 0.581377 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 43284. Total Elements; 27080. Maximum Aspect Ratio; 13.478. % of elements with Aspect Ratio < 3; 99.7. Percentage of elements with Aspect Ratio > 10; 0.0222. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:01. Computer name:. ## Mesh Quality Plots Name; Type; Min; Max. Quality1; Mesh; -; -. blenderaugerlong-Blade analysis long-Quality-Quality1.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -1.97617; -2.95572; 0.000421062; 3.55549. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.00203958; 0.00999769; -0.00583105; 0.0117522. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 1.453e-01N/m^2 Node: 42507; 1.163e+07N/m^2 Node: 41507. blenderaugerlong-Blade analysis long-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 6; 3.127e-01mm Node: 1115. blenderaugerlong-Blade analysis long-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 8.134e-12 Element: 20556; 5.196e-05 Element: 26395. blenderaugerlong-Blade analysis long-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Max von Mises Stress; 1.779e+01 Node: 41507; 1.424e+09 Node: 42507. blenderaugerlong-Blade analysis long-Factor of Safety-Factor of Safety1.

# Conclusion