|  | Simulation of blenderaugershortonly Date: Sunday, January 4, 2026 Designer: Solidworks Study name: blade analysis small Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Interaction Information 5 Mesh information 6 Sensor Details 7 Resultant Forces 7 Beams 7 Study Results 8 Conclusion 10. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: blenderaugershortonly Current Configuration: 默认. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line1; Solid Body; Mass:0.0931969 kg Volume:1.16496e-05 m^3 Density:8,000 kg/m^3 Weight:0.91333 N; C:\Users\User01\Desktop\vffs\design\design\blenderaugershortonly.SLDPRT Jan 4 01:03:50 2026.

# Study Properties Study name; blade analysis small. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\User01\Desktop\vffs\design\design).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; AISI 304. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.06807e+08 N/m^2. Tensile strength:; 5.17017e+08 N/m^2. Elastic modulus:; 1.9e+11 N/m^2. Poisson's ratio:; 0.29. Mass density:; 8,000 kg/m^3. Shear modulus:; 7.5e+10 N/m^2. Thermal expansion coefficient:; 1.8e-05 /Kelvin.; SolidBody 1(Split Line1)(blenderaugershortonly). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-2; Entities:; 3 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 0.00558717; -0.0377856; 3.52518; 3.52538. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 5 face(s). Type:; Normal to selected face. Value:; 2,000. Units:; N/m^2. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Interaction Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 1.47406 mm. Minimum element size; 0.491348 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 49011. Total Elements; 31332. Maximum Aspect Ratio; 10.882. % of elements with Aspect Ratio < 3; 99.9. Percentage of elements with Aspect Ratio > 10; 0.016. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:. ## Mesh Quality Plots Name; Type; Min; Max. Quality1; Mesh; -; -. blenderaugershortonly-blade analysis small-Quality-Quality1.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.00558717; -0.0377856; 3.52518; 3.52538. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -9.35514e-05; 0.000199017; 0.00145948; 0.00147595. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 2.444e+00N/m^2 Node: 19; 9.994e+06N/m^2 Node: 20. blenderaugershortonly-blade analysis small-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 5; 5.659e-02mm Node: 7. blenderaugershortonly-blade analysis small-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 4.931e-11 Element: 5584; 3.566e-05 Element: 7548. blenderaugershortonly-blade analysis small-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Max von Mises Stress; 2.069e+01 Node: 20; 8.463e+07 Node: 19. blenderaugershortonly-blade analysis small-Factor of Safety-Factor of Safety1.

# Conclusion